THE LAND RESOURCE SURVEY OF THE

PANDO DEPARTMENT OF NORTHERN

BOLIVIA.

Thomas T. Cochrane

Sergio Laguna

Martien  A. Beek

DHV Consultants BV,

P.O. Box 1399,

3800 BJ Amersfoort,

The Netherlands.

1994

CONTENTS

SECTION  1.  THE STUDY

1a.   INTRODUCTION AND OBJECTIVES                      PAGE  No                          3	                	1b.   METHODOLOGY                                                                                                6

SECTION  2.  THE SURVEY RESULTS

IIa.   SUMMARIZED RESULTS.                                                                                11

IIb.   SOTER UNIT DESCRIPTIONS                 (SECTION IIb, PAGE Nos.)            24

SECTION  3.   THE LAND USE ZONING MAP

LAND USE ZONES                                                                                                   106

SECTION  4.   SUMMARY AND CONCLUSIONS

SUMMARY                                                               117                                                                                        	CONCLUSIONS                                                                                                              119

REFERENCES                                                                                                121

FIGURES 1, 2 and 3                                                                            138

TABLES  I to VII                                                                                  127

README NOTES FOR PC Users PACKET                                        136

APPENDICES I to V

PC USERs PACKET  (attached)

THE LAND RESOURCE SURVEY OF THE PANDO DEPARTMENT OF 				NORTHERN BOLIVIA.

SECTION I,   THE  STUDY

Ia.   INTRODUCTION AND OBJECTIVES

The study region

The land resource survey of the Pando Department of Northern Bolivia was carried out as an integral part of the "Zonising Project," of Bolivia.  The study region is illustrated by Fig.1, and covers a large part of the northern Amazonian region of tropical Bolivia.

Terms of reference

It was considered that the sustained development of the Pando Department would be enhanced through the carrying out of a digitized Land Resource Study, and the preparation and implementation of a Regional Land Use Zoning  (LUZ), Map, which would be the technical base for proper land use planning and long-term agricultural development planning.  The digitized maps would delineate areas where different land use practices might be encouraged or discouraged, and the accompanying digital database and report would provide the  technical soils information so that improved land use  practices might be stimulated, developed, and researched, and contra productive practices reduced and where possible, eliminated.

The land resource study

The Land Resource Survey was basic to the preparation of the Land Use Zoning Map and Plan;  this involved the mapping of Land Units with homogenous sets of terrain, soil and agro-climate characteristics following the methodology developed by the International Soil Science Society, the World Soils and Terrain Digital Database (SOTER) initiative (ISSS, 1986a,b; ISSS, 1987; Shields and Coote, 1988, 1989; ISRIC 1990a,b) and revised by ISRIC (1991.1993).  The methodology has been accepted as the officially recommended methodology for small scale land resource mapping by the Food and Agricultural Organization of the United Nations (FAO), (Pers. Comm. Dr. W.G. Sombroek, Director, Land and Water Development Division, 1992).  The Map covers the Department of Pando with an average precision approximating  the 1:500,000 scale.  The work is recognized as an integral part of the world-wide SOTER initiative.

The SOTER Mapping Approach

The World Soils and Terrain Digital Database (SOTER) mapping methodology identifies areas of land with  "distinctive, often repetitive patterns of land form, lithology, surface form, slope, parent materials and soil" (ISRIC, 1993).  Effectively, SOTER UNITS delineate areas of similar landscape where the same type of farming or forestry might succeed.  Consequently their careful description is fundamental to land planning and agrotechnology transfer.

Previous Knowledge of the Region - Objective of Present Studies

Literature review and other studies

The FAO-UNESCO soil map of the World (1971, 1974), indicates that the soils of the region are low fertility Ferralsols (Oxisols) and Acrisols (Ultisols), which support a fragile tropical forest ecosystem; the region is inter-tongued with degraded, mainly seasonally wet savannas in its south-eastern sector.  Many authors consider that similar Amazon forest soils are incapable of sustaining agricultural or livestock production after the primary vegetation is removed (Gouru, 1961; Seltzer, 1967; Reis,1972; Tosi,1974; Goodland and Irwin, 1975; Budowski, 1976; Schubart, 1977; Irion, 1978; Goodland et al., 1978).  -Yet there is ample evidence to show that agricultural and livestock production is feasible (Falesi,1972, 1976; Alvim,1978, 1979; Sanchez,1977; Serao et al., 1979; Toledo and Morales,1979; Cochrane and Sanchez, 1982).  The conflicting opinions reflect different concepts how Amazonian soils might be managed, particularly where inputs of fertilizers and amendments such as lime  could be made available economically.   Further, they often fail to recognize that over Amazonia, or any part of it, soils vary considerably and have inherently different fertility characteristics.

A more detailed appraisal of the study region was carried out by Cochrane(1973), unfortunately without the aid of modern satellite imagery and remote sensing techniques. He found that many of the soils in the region were highly weathered and of low natural fertility.  Further work over the region confirmed the earlier findings (Cochrane et al., 1985).

Much more recently, Cochrane et al.( 1992), carried out a study of the land use potential of the Brazil Nut region of Northern Bolivia, following the SOTER methodology as detailed by ISRIC (1991). This latter study delineated provisional SOTER units of the Pando Department  and provided details of the inherently fragile nature of the soils of the region.  It emphasized the very real hazards of indiscriminate land usage in terms of the medium and long term productivity and prosperity of the region.

Localized studies

Localized studies of limited areas of the region include the work of:  Iriate et al. (1976) between Riberalta and Guayaramerin; Carrera (1976) near Riberalta; Sainz et al. (1984) in the vicinity of Porvenir in the Nicolas Suarez Province; Iporre (1984) in VacaDiez Province; Macias (1984) north of Chive near the border with Peru; Iporre (1985a) in the vicinity of Santa Rosa de Abuna; Iporre (1985b) in the Federico Roman Province; Macias (1985) in the northern Ballivian Province; the Servicio Nacional de Aerofotogrametria, SNA (1978) in north-western Pando; studies by Conet (1978a,b) in the Cobija-Puerto Heath region; CONSA and HARZA (1983) in the immediate Puerto Heath region; and the study of Salm and Marconi eds. (1992) over the Manuripi -Heath National Amazonian Reserve.

It is of significance that no study to date has identified any substantial areas of fertile soils that are not subjected to seasonal flooding and, or, have poor drainage conditions.  They invariably paint a picture of inherently poor soils supporting  fragile forest ecosystems, although it is equally clear that considerable differences in land formations and soils exist over the region.

The present studies.

The present studies are a follow up of the studies carried out by Cochrane et al. (1992), of the land use potential of the Brazil Nut Region of Northern Bolivia following the preliminary SOTER methodology as detailed by ISRIC (1991).  They have been designed to provide a systematic, more in-depth appraisal of the varying soils found in the Pando department, not only as a basis for territorial zoning  but also to determine the most economically and ecologically sound uses for the land.  The studies follow the latest SOTER methodology (ISRIC-FAO-UNESCO-ISSS, 1993), to provide a computerized database of the land and forestry resources of the Department.

Ib.    METHODOLOGY

The SOTER methodology

The SOTER methodology adopted for studying the region, is designed to facilitate the speedy but careful, systematic appraisal of its land resources, through the compilation of a computerized database of both new information and existing information, where the latter is available.  The methodology essentially follows that described in the  Procedures Manual (ISRIC,1993), although additional flexibility was added to the system to better cope with larger scale studies, and  to facilitate use by Personal Computers of varying capacities using different software programs.  The study was completely computerized.

Climatic studies

Work started with an examination and analysis of the available meteorological data, followed by its incorporation into the SOTER database.   A considerable amount of climatic work has been carried out  for the region in recent years by Hargreaves and his co-workers or associates(Hargreaves, 1977; Hancock et al, 1979; Hargreaves et al, 1978; Cochrane et al,1985 and Povoa, 1988).  Hargreaves' methodology for calculating  potential evapotranspiration was chosen as it gives a good correlation with the measured lysimeter data collected under the control of the American Society of Agricultural Engineers, ASAE (Hargreaves,1977, 1989).

Delineation of SOTER UNITS on satellite imagery

The initial, provisional delineation of the land (SOTER) units was carried out on composite satellite imagery prepared by DHV's Geo-informatics Division,  using bands 3, 4 and 5 of the TM, 1:250,000 imagery; refer to  the  SOTER UNIT MAP included with the PC Users packet.  At the same time, the existing  soils information was reviewed.

The FAO-ISRIC soil profile database

The recorded soil profile data was reorganized using the FAO-ISRIC  Soil Database (SDB2 version, 1991), to facilitate the, storage, retrieval and examination of that data.   Following field work, the in-field descriptions of the soil profiles examined by the authors were also added to this database.  In fact, the final soil profile database consisted of over 400 soil profile descriptions and their analyses, more than 300 of which  were described during the course of the in-field studies between 1992 and 1993.   All the soil profile descriptions were standardized according to the latest FAO guidelines (FAO, 199O).  Appendix I lists these profiles and their locations, together with print-outs of the typical reference pedons considered an integral part of the SOTER database.  Apppendix ll records the modifications made to he FAO-ISRIC Soil Database to describe the soil attributes according to the FAO, 1990, guidelines.

Soil analysis

The soil profile samples were analyzed in the CIAT-BTAM (Centro de Investigaciones en Agricultura Tropical - British Tropical Agricultural Mission), soils laboratory in Santa Cruz, under the supervision of the senior author.  The methodology used for the analysis is detailed by Cochrane and Barber (1993);  for practical purposes it is similar to that of the International Soil Reference and Information Center ISRIC  (van Reeuwijk, 1987).  The quality of the analyses in the CIAT-BTAM soils laboratory is monitored bi-annually by the international system set-up by ISRIC for comparing soil analyses world-wide, which is currently supported by the University of Wageningen, Holland.

Field work

Because of the lack of comprehensive soil and field information due to  the  area specific nature of most of the recorded information on the region, it was necessary to carry out a series of in-field studies to describe the soil and land characteristics and collect representative soil profile descriptions and horizon samples.  Four major field expeditions were carried out during 1992 and three in 1993, and these were complemented with several smaller expeditions, to provide a representative coverage of the region.  Aerial reconnaissance completed the task of examining the most inaccessible parts of the region, and helped with the extrapolation of the on-ground studies.  The latter was carried out by the senior author (Dr. Thomas T. Cochrane), piloting a Piper PA-18, STOL aircraft, specially adapted for aerial reconnaissance work.

The major soil study expeditions followed the regions of influence of the Beni,  Acre,  Orthon, and Madre de Dios rivers, and transected the region from west to east and  north to south.  Soils were studied and sampled along many transects, mainly following exiting bush trails both by on foot and by motor cycles.   Hand-carried Geographic Positing System GPS, equipment was used to locate the position of sample sites and the study transects, and to identify these on the satellite imagery.

Map making,  map digitization and GIS

With the successive completion of the field work and the laboratory analysis of soil sample data, (and following the climate studies of the region), the definitive Land (SOTER) Units were demarcated directly on the 1:250,000 satellite imagery, and their boundaries digitized using geographic information system GIS, software.  The IDRISI software developed by Eastman (version 4.1, 1993), of Clark University, U.S.A., was chosen for this purpose;  it is the GIS software currently being used by the United Nations Environmental Program, UNEP.

Input of the mapped information into the IDRISI system was carried out using the IDRISI ancillary program TOSCA, developed by Jones (1991), also of Clark University.  A small "Summagraphics" digitizing tablet, Summasketch II, was the chosen hardware used for this purpose.   Once transferred to IDRISI, the mapped SOTER UNITS were subjected to the ample spatial analysis procedures of that system, and formed a basis for computer generated thematic maps.  IDRISI provides for the export of map information in a variety of formats compatible with all the major GIS, commercially available software.

The final SOTER UNIT map was prepared  in electronic format, and may readily be converted for use with other Geographic Information System GIS, software, such as ArcInfo and ERDAS.  Files prepared in the latter format have been included  with the PC Users Packet appended with this report.

The SOTER digital database

Concurrent with the definition of the SOTER UNITS, the SOTER  land resource database file set detailed in the SOTER manual (ISRIC, 1993),  was organized into a relational database management system RDBMS,  Fig. 2.  Appendix II records the SOTER file set actually set up for the study, along with observations on the minor differences and additions to the manual.  It might be noted that SOTER uses a set of files compatible with dBASE 4+;  this same file structure, with some minor modifications, was used with Paradox 4+ (both WINDOWS and DOS versions), for the present study, but the files can also be used with dBASE 4+, and the specialized software package currently being developed for SOTER by ISRIC.  Consequently, the database is in fact the first  SOTER database to be setup up in strict accordance with the FAO -ISRIC -UNEP -ISSS 1993, standards.

It should be noted that an additional file "PROF.GRP" (soil profile groups), was added to the SOTER database file set to group similar soils into groups to facilitate the statistical analysis of the spatial variation, or the variation over an area of terrain, of soil attributes, especially their chemical (nutritional) properties.

The objective of the SOTER database file set, apart from facilitating computerization, is to provide a facility for describing land resource information on an area or spatial basis, and  to transcribe the point (soil profile) data from in-field observations to the mapped areas shown on the SOTER map.   Soil profile data per se', is incorporated into the system via the FAO-ISRIC (1991),  SDB2 software.   It should be emphasized that the design of the SOTER database file system is open-ended, and facilitates the integration of other types of databases  including socio-economic data, and specific forestry and agronomic data,  with the land resource database.  It is linked, to and from, with  Geographic Information System GIS, software, such as IDRISI in the case of the present study,  as is illustrated by Fig. 3.

The prize-winning "PARADOX " RDBMS, developed by Borland International Inc., (the company which acquired dBASE 1n 1992, the venerable although somewhat dated industry standard, database software), was chosen for managing the SOTER files (the structures for which , as already noted, were originally compiled in dBASE format by FAO-ISRIC),  as it is user friendly, very comprehensive and facilitates the input and export of data files from most other commercially available software packages.  The  DOS version of the program was in turn complemented with DESKTOP for PARADOX 4, a very useful supplement for the DOS version of  PARADOX, developed by  Kallista Inc.  It should be added that the spreadsheet program Quattro Pro (version 5.0 for Windows), also by Borland International Inc., was used to supplement the systems statistical analytical capacity, and that other programs including Microsoft WINDOWS, Norton Utilities, and anti virus and compacting programs, were used as additional tools to speed-up the flow of work.

The computerized system

Figure 3, is a flow chart that  illustrates the basic computer system set-up.    It is important to note that the entire system is open, and  uses inexpensive, commercially available software packages that are backed with strong support organizations providing regular updates and improvements to their programs.  Perhaps the only weak part of the system  at the present time is the FAO-ISRIC   Soil Database (SDB2), which has some inbuilt weaknesses, particularly in terms of the reporting of soil analytical data and general flexibility and reliability; but even this software permits the transfer of files as dBASE files that are readily imported into PARADOX and IDRISI, for alternative processing.

Effectively, the system interlinks the relational database management  of the SOTER file set (by PARADOX and, or dBASE), with the Geographic Information System GIS, IDRISI  for map making, spatial analysis and satellite image interpretation.

The land resource files of the study region that were compiled for the system,  and that summarize all the information gathered during the course of the survey work, have been recorded as a  PC Users Packet, attached to the report.  The latter has been subdivided into 3 Sections:

1) SOTER files for PARADOX or dBASE;

2) IDRISI files; and

3) FAO-ISRIC, SDB2 files.

Only IDRISI IV software (cost about $US100), and PARADOX 4.+ (cost about $US180), are needed to set up and run the system, which is suitable for most personal computers.   Additional information has been recorded in the "README" notes accompanying the Users Packet.

SECTION II

THE SURVEY RESULTS

Introduction and Orientation.

Section IIa which follows, summarizes the major land resource characteristics of the region.  Written notes on the land resources of the mapped SOTER UNITS are recorded in Section IIb;    the detailed information collected as a result of the survey is recorded according to the SOTER digital database methodology in the PC User's packet, attached.  The latter is subdivided into three sections:  Section 1, SOTER files; Section 2, IDRISI files and Section 3, FAO-ISRIC  SDB (Soil Profile Database files).  (Refer Figs.2 and 3, and Appendices l to lll for further details.)

SECTION IIa, SUMMARIZED RESULTS.

CLIMATE

The Equatorial Trough and Polar Air Masses

The major determinant of the climate of the region is the seasonal migratory movements of the low pressure Equatorial Trough ET, often referred to as the Inter tropical Convergence Zone.  The position of the ET follows the seasonal march of the sun, but lags behind by about 2 months.  During the southern hemisphere summer a continental heat low pressure zone develops over northern Argentina, Paraguay, and southern Bolivia which influences the ET movement, and results in high instability and heavy rains throughout the region during the December to March period.

In contrast with the warm northerly air masses, invasions of cold polar air masses are common during the dry winter months, and produce marked and rapid drops in temperature as the cold front passes northward.  These cool winds, referred to as "surazos" in Bolivia, may last from 3 to 8 days or, in exceptional cases, up to 15 days. Temperatures may drop to as low as 6 degrees Centigrade in a matter of a few hours.  The passage of the cold front is often accompanied by rain squalls and marked atmospheric turbulence, especially during the April to Aug. period.

Meteorological data analysis

Data from 5 meteorological stations, Cobija, Guayaramerim, Puerto Maldonaldo (Peru), and Rurrenabaque was compiled as an integral part of the SOTER  database, and is recorded as Table I.  These were originally compiled by Hargreaves and his coworkers (Cochrane et al., 1985).  The data recorded and calculated include:

TEMP          -mean temperature in degrees Celsius

SUNP          -percentage of possible sunshine

RADI           -mean solar radiation, Langleys per day

RAIN           -mean precipitation, in mm.

PETH          -potential evapotranspiration, in mm.

RDEF          -precipitation deficit, in millimeters

RR75          -dependable precipitation, 75% prob.

MAI             -moisture availability index

MAI is a moisture adequacy index at the 75% probability level of precipitation occurrence, that was introduced into the literature by Hargreaves(1972), to identify soil water adequacies.  Hargreaves showed that there is a good relationship between MAI and crop production when soil moisture is adequate for a week or more and recommended a level "less than 0.34" to define a dry month.  A wet month then, is defined as on with a MAI greater than 0.33, bearing in mind that this level may be too low for soils with very low moisture-holding capacities.

The dry season

An examination of the meteorological data sets indicates that the region has a distinct dry season, ranging from 3 to 5 months, and that there is a  trend for the dry season to be a little more pronounced in the eastern part of the region.  This has been confirmed by the work of Newel, recorded by van Wambeke (1981), on determining soil moisture regimes for those stations; assuming a soil with a medium moisture holding capacity, only Cobija and Rurrenabaque classify as having an udic soil moisture regime, signifying less than 90 accumulative days per year with dry soil conditions.  In fact, Newel indicates that Cobija has a "dry" udic soil moisture regime.

Potential evapotranspiration

Potential evapotranspiration PETH, was calculated to determine the water balance and growing seasons;  an accurate estimate of potential evapotranspiration provides a most useful parameter to help judge the agricultural and forestry potential of an area.  For any given month, providing that MAI  and the soil moisture holding conditions are high enough to allow unrestricted water availability, and temperatures do not fall below limits tolerated by specific crops, then the potential evapotranspiration level also provides a guide as to the amount of energy available for plant growth.   In fact Cochrane and Jones (1981), showed that natural physiognomic vegetation patterns throughout tropical America, including Amazonia, follow their wet season potential evapotranspiration regimes, always providing soil drainage circumstances are adequate.

The growing season

Not taking to account soil conditions, it is clear that the region has an adequate growing season for many  tropical crops, including  perennial crops that can withstand a definite, but not too prolonged, dry season.  The exceptions are crops sensitive to sporadic cold spells, including some rice and corn cultivars that agronomists might be tempted to grow under irrigated conditions during the dry winter months.  Clearly, perennial crops sensitive to significant dry season conditions, such as many African Oil Palm cultivars, may not yield well, except on soils with high water retention properties or mild soil internal drainage problems.

The compilation of the SOTER climate database files provides a useful source of information for research workers to carry out in-depth analyses of the climatic conditions of the region,  for the introduction of new crop and tree-crop cultivars.  It provided the present workers with an additional guide as to the probable soil moisture conditions of the separately mapped SOTER UNITS, and a more accurate guide as to their agricultural, forestry and animal production potentials.

Focusing on soil conditions

As climate is not a major factor limiting the growth of selected crops and their cultivars, the appraisal of the land resources of the region must focus on its soil conditions; firstly in terms of land form and topography, soil drainage and, or flooding problems and soil moisture holding capacities.  Then, other soil physical conditions limiting plant growth such as soil depth, stoniness and the presence of root restricting horizons or layers must be taken to account.   Finally, soil chemical conditions including both nutrient conditions and potential toxicity conditions must be analyzed.

LANDSCAPES

Land form, Lithology, Hydrology and Vegetation

The region covered by the SOTER  UNIT  map extends from Peru in the west and is frontiered with Brazil to the north and east.  To the south, and more so to the south-east, it merges with the Bolivian Mojos savannas, locally called "pampas".  This latter area is rich in Pre-Colombian archeological earthworks, undoubtedly of agricultural origin from lost civilizations.

Major rivers

The region is segmented by several major rivers.  From the border with Peru in the west, the mighty Madre de Dios river runs eastwards until it joins the northward flowing Beni river near the township of Riberalta.  Further to the north, the Orthon and its major tributaries, the Madidi and Tahuamanu rivers, also flows approximately eastwards  until it to joins the Beni river.  Along the frontier with Brazil, first  the Acre river, and then the Abuna, channel eastwards, the latter to meet the north running  Mamore river, which has in turn, merged with the Beni river a little north of  Cachuela Esperanza, a township known for its Pre-Cambrian, granitic rapids that impede navigation to the Atlantic Ocean.  There are wide seasonal fluctuations in the volume of water flow in all the rivers, resulting in considerable differences in water levels, often in excess of 15m, and consequent seasonal flooding of their younger alluvial deposits.

Lithology and rivers

It is safe speculation to assume a relationship between the direction of flow of the major rivers of the region, and its over-all geology and land form.   The region is essentially a dissected plains area.  However in the west it is higher, with a median altitude in excess of 150m; Quaternary fluvial sandstone-mud stone mixed and often compacted sediments overlie Tertiary mixed sediments (Leyton and Pacheco, 1989).  These appear to have been uplifted in relatively recent times, and subjected to more intensive natural erosion, resulting in strong peneplanation and significant areas of small hills.  The higher formation gradually and unevenly slopes towards the east. About midway across the region, median altitudes fall to less than 150m; here the lithology is probably almost exclusively of Quaternary fluvial origin, and peneplanation much less pronounced.

The Brazilian shield

The northward flow of the Beni and Mamore rivers approximately coincides with the influence of the Brazilian Pre-Cambrian shield formation (Ahlfeld and Branisa, 1960), which underlies the Quaternary fluvial sediments of the eastern  segment of the region.   Small protuberances of the Brazilian Shield, including small hills formed from granites and gneiss, may be seen near Cachuela Esperanza and northwards to the border with Brazil.  The rapids along the Beni and Mamore rivers from Cahuela Esperanza northwards, are a direct consequence of  the Shield formation, and indicate the shallowness of the formation along the border region with Brazil.  Extensive areas of low-lying , seasonally water-logged lands are found in this area, possibly reflecting the underlying influence of impermeable Shield materials, although more probably a result of the effect of the Shield in "damming" surface and ground water flows.

Land form

The Land form Map generated in IDRISI as a part of the SOTER  digital database of the study (PC  Users packet attached), provides an overview of the land form of the region in terms of : Hills, High Peneplains, Low Peneplains, Plains and Flood plains.  Table II gives a breakdown of these regions in terms of areas, and the areas of dissected or sloping lands within those major formations.  Details in terms of SOTER UNITS are recorded in section IIb, as written descriptions of the SOTER  UNITS.  Clearly, a wealth of  additional information can be drawn from the computerized SOTER database in the PC Users packet attached.

Other Major Landscape Features

To emphasize the diversity of the lands of the region, a SOTER Landscape map was generated by subdividing the land form map according to overall soil drainage conditions and native forest versus savanna vegetative cover (see PC Users Packet attached, Section 2, IDRISI Files).    Table III provides a short summary; it emphasizes that a significant proportion of the region has soil drainage problems in addition to the generally recognized flooding problems; a third of the land surface is affected by these two problems

Physiognomic native vegetation

The native vegetation of the region has been described in terms of the physiognomic classes used by UNESCO (1973), with the modifications  summarized in Table IV.  A map of the native vegetation of the region (SOTER Native Vegetation Classes) in terms of compound classes,  has been recorded in Section 2, IDRISI files, of the PC Users packet, attached. (This map should not be confused with the Economic Vegetation Map prepared as a parallel activity to the studies.)

Tropical Semi-evergreen Seasonal Forests predominate through the region, with hydroseres found as inclusions on poorly drained or flooded lands.  The forests on the well drained lands have emergents forming an open upper story canopy that ranges from 35 to 50m in height, depending on the specific land areas.  Most of these emergents loose their leaves during the latter part of the wet season (August -September), including the Brazil Nut trees (Bertholia Excelsia), and Rubber (Hevea Brasiliensis).  In fact, the Brazil Nut producers of the region estimate the relative yields of the following year's crop (February-March period), by judging the numbers of Brazil Nut "cocos" on the trees at that time of the year; this influences the prices paid to collectors by the processors.

Savanna lands

There are some minor areas of mainly poorly drained savanna lands in the south-eastern areas of the region, about 19,000ha in all.   The boundary between the forests and savannas is quite abrupt, with transitions occurring over distances of 100-200m or less, which reflect the radical changes in soil physical and sometimes chemical conditions, as seen elsewhere in the sub-continent by the senior author (Cochrane, 1989).

Native vegetation and soil conditions

There is a strong correlation between the native vegetation and soil physical conditions throughout the region.  This is evidenced from Table V, which summarizes the cross tabulation of the SOTER Native Vegetation map with the SOTER Landscape Map  that emphasizes soil drainage and flooding conditions,  both found in the PC Users package, Section 2, IDRISI files, attached.  Consequently, the importance of the soil studies of the region, especially in terms of the probable productivity of native forests for both extractive and wood products, apart from its agricultural and minor pastoral potential, cannot be over-emphasized.

SOILS

Soil Geography

Thematic map making

In the SOTER methodology, Terrain Components with their varying soil patterns, bridge the gap between the mapable SOTER  Units and soils, although with a larger scale study like the present, some  mapping of Terrain Components themselves was possible.

In making a thematic map at  a scale of 1: 250,000 or smaller, whether to provide a picture of the region according to  established classification systems of vegetation, land use and soils, or in terms of specific soil properties such as topography, drainage, stoniness, fertility and so on, unless it is otherwise stated, the map, is a compiled as follows:    a Terrain Component is assigned the classification of its principal soil or soil property, then the SOTER UNIT, when only such have been mapped as a whole, is assigned the soil or soil property classification of its principal Terrain Component.  The mapped units thus classified, are grouped by the GIS software program to form the thematic map.

The resulting maps are comparable to soil and other thematic maps produced using more traditional methods, as the latter invariably represent generalizations caused by scale considerations.  Unfortunately scale limitations are often ignored in examining and interpreting maps.

The SOTER database

In contrast to traditional soil mapping, the SOTER methodology facilitates a much more in-depth examination of soils and their properties in statistical terms, as the basic information about each SOTER Unit is compiled in a series of computerized database files (refer Fig.2).  As a consequence, the methodology facilitates the compilation of accurate statistical information about soil properties, including the aerial extent of such.  Following analysis, aggregated or compounded information can be depicted as thematic maps for illustration purposes.  In addition, information from satellite imagery can be analyzed via GIS, and integrated directly with the SOTER database.

Soil classification

As a part of the SOTER study, soils were classified according to both the FAO-UNESCO  revised legend (1990), and the latest  United States Department of Agriculture's Soil Taxonomy classification (1987).   Table VI provides a summary of the region classified according to both these systems, and soil classification maps of both systems indicating the geographical distribution of the soils have been recorded in Section 2, IDRISI files of the PC Users packet, attached.  As noted from Table VI, 82% of the soils classify as Ferrasols or Lixisols according to the FAO system and 58 %, classify as Oxisols and 24% as Ultisols, according to Soil Taxonomy.   This indicates that the majority of the soils of the region are highly weathered soils poorly supplied with plant nutrients.

It should be noted with reference to  Soil Taxonomy, that questions can be raised as to whether some of the soils of the region would classify as Oxisols or Ultisols according to the exactly defined textural limitations specified by that system.  The problem arises because the authors of Soil Taxonomy do not specify how the soil textural analysis should be carried out.   -As indicated by the American Society of Agronomy's "Methods of Soil Analysis, 1987", clay analysis may vary as much as 2 to 4 times  depending on the type of pre treatment used, including such procedures as the removal or otherwise of organic matter, deferralization and so on.   Thus, whilst generalized soil characteristics can be inferred from soil classification systems, criteria used for separating classes are often somewhat dubious.  Clearly, the SOTER  database facilitates a much more in-depth understanding of soils, particularly of their physical and chemical characteristics, as the way their analyses are carried out are referenced.

Soil Physical Characteristics

Soil physical characteristics were classified and coded in terms of slope, depth, drainage, temperature regime, moisture regime, moisture holding capacity, texture, presence of coarse materials including gravel and stones, presence of root restricting horizons and other parameters as detailed as part of the SOTER database.    The examination  of soils according to their physical properties facilitate the evaluation of their susceptibility to erosion and their suitability for agricultural and tree crop production from a physical standpoint.   In the words of the late F. Hardy (Pers. comm. with senior author), to study the "root room" of a soil, or its physical characteristics as a plant growth environment.

Erosion Hazard

Table II provides a summary of the region's land forms.   It should be noted that apart from the specific hill and dissected plain formations, the peneplain landforms which cover 56% of the region, have varying grades of dissection by streams and rivers, resulting in significant areas of steep lands.  Consequently, over 30% of the region is covered by steeply sloping lands that are very susceptible to erosion; only their existing forest cover protects these lands from severe soil loss.  In fact, many of the hilly lands are doubly susceptible to erosion, as their soils have a distinct textural change (Ultisols with light textured topsoil horizons over heavier textured subsoil horizons).   Even on relatively gentle sloping sites, such soils are susceptible to erosion, as may be witnessed by the many eroded areas of lands recently cleared of their forests  for cattle production in the vicinity of Cobija.

Poor drainage and flooding problems

From the SOTER  database  it is estimated that approximately 32%,or 2,024,000 ha of the region is covered by soils that are either poorly drained and, or, are subject to annual flooding.  In fact, 825,700 ha of the non-flooded soils have drainage problems (Refer  Section 2, IDRISI files, PC Users Packet, attached, for the SOTER  Landscape Map of the region to indicate the distribution of these soil problems in the region).   The latter problem is region specific, and predominates in certain SOTER units.  Clearly the magnitude of the flooding and especially the soil drainage  problems imposes a severe limitation on land usage throughout the region.

Soil moisture holding capacities

The soil textural  characteristics of the region have been recorded in the SOTER database.   Although a lesser proportion of the region (approx 34%) has light textured soils or soils with lighter textured topsoils, it should be noted that the heavier (clay) textured soils have high Al-Fe sesquioxide levels which reduces their capacity to withhold soil moisture, and so  they tend to act more like light textured soils in this respect.

Climate-wise it is clear that most of the region suffers from seasonal moisture stress during 3 to 4 months of the year, (see notes on climate, above); consequently, the low soil water holding capacities of sandy soils, or of the soils with lighter textured topsoils, aggravate this problem, especially for the growth of shallow rooted crops.

Coarse soil materials

Hardened plinthite or laterite are seen occasionally, generally in geomorphically predictable positions in the landscape, particularly on the edges of the peneplains. However, such formations are not considered a serious restriction to plant growth, due to there generally limited extent.  Notwithstanding, a few of the SOTER Units have shallow soils where plinthite, and or hardened plinthite may be a serious limiting factor to plant growth; most of the savanna soils have this problem.

A more common problem with the soils of the region, is the presence of gravel size laterite particles and, or Fe-Mn concretionary materials which makes hand cultivation difficult in many instances.

Soil Chemical Properties

Soil analyses were carried out on most of the profile samples collected during the field expeditions,  a lesser amount of analyses have been recorded in the literature.  This data has been summarized  in Section 1, SOTER files of the PC User's packet, as interchangeable PARADOX - dBASE - Quattro Pro files and in Section 3 of the same packet as an integral part of the FAO-ISRIC  soil profile database.

The Fertility of the Non-hilly, Non -flooded, Well Drained Lands.

The following notes provide a brief overview of the major soil chemical properties affecting plant nutrition, or the "fertility", of the non-flooded, adequately drained, non-hilly or steep (less than 8% slopes), lands of the region, which might be considered as possible lands for agricultural and, or  pastoral production, according to their "gross" physical characteristics.  Lands so defined, cover approx. 52% of the total study area or 3,308,762 ha out of the total area of  the department, of approx. 6,318,900 ha  The information has been drawn from the SOTER database, and is summarized as Table VII.   It provides an example of the type of information that can be extracted from the database.

Soil acidity or pH:

About 79% of the topsoils (SOTER horizon No.1), and 70% of the subsoils (SOTER horizon No.3), have low, pH levels (less than 5.4, regarded as strong acid conditions), which could affect the availability of some essential plant elements, and would provide the necessary soil solution environment for Al toxicity problems (Cochrane et al.,1981.).  It is interesting to note, that in many soils the subsoil pH levels are a little higher than the topsoil levels, a positive condition where this occurs.

Carbon levels:

As indicated by Table VII, the carbon (C), levels, which are used to indicate soil organic matter contents, are in the medium range in the topsoils but very low in the subsoils.  As soil organic matter levels are intimately linked to soil fertility, there is clearly a need to avoid the removal of the upper horizon of these soils.

Phosphorus:

The available phosphorus (P), levels of the topsoils are predominantly,  in the lower medium range, and unlikely to sustain crop growth for any length of time.

Exchangeable cations; calcium, magnesium, potassium and sodium:

With the exception of the very low sodium levels, the topsoils have exchangeable cation levels in the medium to low range; however, subsoil levels are low.  Levels follow the organic matter pattern, which suggests that the soils are highly leached and intrinsically poor;  they indicate that soil fertility is largely dependent on the organic matter cycle of the forest cover.

Effective cation exchange capacity:

The effective cation exchange capacity of these soils, even in their topsoils (70% rated as low), is predominantly low. This clearly indicates that they have been formed in strongly weathered parent materials, and that they have an inferior capacity to hold and supply nutrients for crop growth.  Anion exchange capacity levels were not measured for these soils, although it is likely that  such could play an important role in the retention of plant nutrients, especially with respect to nitrogen cycling (Cochrane and Souza, 1985).

Aluminum toxicity:

Table VII shows that 70% of the topsoils and 77% of the subsoils of these lands have high percent aluminum Al, saturation levels of their effective cation exchange capacities.   As most of the soils have low pH levels (less than 5.4, a condition permitting the solution of toxic Al hydroxides), they have potentially  toxic conditions for many of the higher yielding tropical crops with low tolerances to high soil Al levels.  Very substantial applications of lime (1 to several tons per ha), would be needed to overcome this condition for many tropical crops.

Other plant nutrients:

It should be noted that there was no analytical data for sulfur or trace elements.  Sulfur levels probably follow the organic matter levels, and deficiencies could well develop following deforestation.  Further, it is quite possible that trace element deficiencies including zinc and possibly boron could occur in these soils.

Summary of the soil chemical conditions.

It is clear from the analytical data  that  almost all of the soils that might be considered for possible agricultural and pastoral uses according to their overall physical conditions, have intrinsically poor and fragile fertility states.  Considering  their parent materials, especially the sandstone influence of the higher, older peneplains, the soils are mineralogically and thus intrinsically poor.  The little fertility that they  have is intimately linked to the carbon cycle of their forest cover. Indiscriminate forest removal  can only lead to the rapid chemical degradation of these lands, as is evident in the Cobija region where lands are being cleared for ranching. Moreover, the predominantly low pH and high Al levels which characterize many of these soils, indicate potentially severe toxicity problems, which can only be overcome by heavy and repeated applications of expensive soil amendments.

The implication of the studies on soil fertility is, that without the availability of cheap supplies of lime and fertilizers, land settlement or "colonization" for agricultural or agro-pastoral purposes, can only lead to an ever-accelerating pattern of deforestation followed by soil chemical degradation, further deforestation and so on leading to eventual environmental disaster.

Finally it should be noted, that while soil fertility does vary in degree from SOTER UNIT to SOTER UNIT, the study did not lead to the discovery of any substantial areas of land that could be considered as being "inherently fertile" and suitable for agricultural and, or agro-pastoral purposes, as is the case of lands surveyed by the senior author in other parts of the Bolivian lowlands (Cochrane, 1973).

Land Use.

Present land use was classified for the region according to the SOTER Land Use Classes, which were adapted from Remmelzwaal (1990).  The following classes were used for the present study  (the codes follow the SOTER coding system, which permits the addition of extra classes to better describe local land use conditions):

AA1 ... shifting cultivation

EV1 ... Brazil nut collection

EV2 ... rubber tapping

EV3 ... both Brazil nut and rubber collection

EV4 ... Brazil nut and rubber collection and selective 			             		felling of timber

EV5 ... Brazil nut collection and selective felling

EV6 ... rubber tapping and selective felling

EH ..... hunting and fishing

FN1 ... selective felling of timber

FN2 ... clear felling of forest

FN3 ... regenerating forest, early stages (<7 years)

FN4 ... regenerating forest, late stages (>7 years)

HE3 ... extensive ranching

HI7 .... intensive grazing animal production on

cultivated pastures

PN1 ... reserves

S......... settlement and mining  (SP, petroleum)

From an analysis of the SOTER database (refer PC Users packet, appended), it was seen that 5,042,600 ha, are presently used for Brazil nut and rubber extraction with varying degrees of selective felling for the extraction of valuable timber species.  Of this area 3,643,961 ha are used  for Brazil nut extraction, and 1,398,639 ha  are used virtually solely for rubber  tapping.    However, with the recent-past collapse in the price of natural rubber, only a small part of the rubber in the region is being tapped.  Further, it is evident that only a lesser part of the total Brazil nut production of the forests is collected.

Other minor uses of the land include: extensive ranching on the natural pampas found in the south-east sector of the region (19,000 ha), some more intensive ranching mainly in the vicinity of Cobija (about 51,000 ha), shifting agriculture for local food supplies (approximately 116,000 ha affected throughout the department, about  66% of which is reverting back to forest, and at varies stages of regeneration), a small area of 1800ha that may be affected by petroleum drilling in the near future, and a more extensive area of lands suitable only for hunting and fishing.

The Land Use Map appended with the report, and included as an integral part of the SOTER database, Section 1, PC Users packet, appended, provides a picture of the various principal compounded land use types over the region, as the scale of the map does not permit greater detail. Nevertheless, the SOTER database files, PRIM_LU and SEC_LU, provide much more detailed information concerning present land use, down to the level of the SOTER  Terrain Components, and provide a means of examining such in the light of the other land resource, forestry and socio-economic information.

SECTION 11b

SOTER - UNIT DESCRIPTIONS

October 1993

SOTER UNIT:  BO-0001

LOCATION:	The unit is found in the south eastern part of the Pando Department; close to Riberalta.

AREA:	29628 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 3 Terrain Components, BO-0001.1 (70%), BO-0001.2 (25%) and BO-0001.3 (5%); these represent the flat well drained areas of the peneplain, the less well drained areas and strips along stream lines, respectively.

ALTITUDE:	120-140 m; median altitude 130 m.

LITHOLOGY:    Siltstones, mudstones, claystones of Quaternary origin, (SC3Q).

HYDROLOGY: The unit is drained by a number of minor streams.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to Sept., during the cool season when cold southerly fronts or "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils of the peneplain have clay textures, yellowish-red topsoils 		and red subsoils.  (PAN-001, Representative Profile BO0001).

Soils with somewhat impeded drainage are found in slightly concave positions of the peneplain, represented by Terrain Component BO-0001.2. These often have clay textures, strong brown coloured topsoils and yellowish-red subsoils.  Small ironstone (Fe-Mn) concretions are often present in the subsoil (PAN-002, representative profile BO0178).

Running parallel along the streams, a sequence or "catena" of soils is found, with well drained soils similar to SOTER Profile BO_0001 on the sloping sites and poorly drained soils on the flats near the stream beds. The latter poorly drained soils often have loamy textures, pale-brown topsoils and a light grey subsoils.  Mottles and concretions may be present. (PAN-003, representative profile BO0503).

SOIL FERTILITY: With the exception of very minor areas of alluviums, the soils are poorly supplied with plant nutrients, have low exchangeable cation capacities and high Al levels. pH levels are low.

Topsoil Analyses:

Terrain Component BO-0001.1, Mean soil analytical data BO0001: pH, ECEC, Ca, Mg, K, Na and P low; Al high.

Terrain Component BO-0001.2, Mean soil analytical data BO0178: pH, ECEC, Ca, Mg, K, Na and P low; Al high.

Terrain Component BO-0001.3, Mean soil analytical data BO0503: pH, ECEC, Ca, Mg, K, Na and P low; Al high.

VEGETATION: Medium to high tropical semi-evergreen seasonal forest, emergent height 20-35 m, covers the unit; Brazil nut trees are present, but nearly 20% have been cut down.

FORESTRY PRODUCTION: Brazil nut collection is still taking place, but the unit is affected by extensive forest clearing.

AGRICULTURAL AND ANIMAL PRODUCTION:  Because of its nearness to Riberalta, shifting agriculture dominates this unit.  There is some minor cattle production.

SOTER UNIT:  BO-0002

LOCATION:	The unit, the Madre de Dios river and its floodplains, transects the region from west to east (see photo 1,2,3).

AREA:	408844 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This floodplain unit has been subdivided into 2 Terrain Components BO-0002.1 (25%) and BO-0002.2 (60%), which represent the higher better drained terraces and the lower poorly drained parts. The total water surface amounts to 15%.  The floodplain varies from 6-20 km. wide.

ALTITUDE:	100-180, median altitude 130 m.

LITHOLOGY:    Recent unconsolidated fluvial sediments, (UFR).

HYDROLOGY: The Sena river and the Asunta, Toromona and Canada streams are the most important tributaries of the Madre de Dios river.  This river is navigable by small craft.  However, when the river has its lowest level towards the end of August, sand banks and some rapids "cachuelas", makes navigation difficult.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to Sept., during the cool season when cold southerly fronts or "surazos", alternate with warm northerly winds.

SOILS:		The soils on the higher, imperfect drained floodplain terraces have silty clay textures and brown topsoils and dark greyish brown subsoils (PAN-004, representative profile BO0612).

The soils on the lower poorly drained parts have clay textures, greyish brown 			topsoils and grey or dark yellowish brown subsoils.  Many mottles can be found               		throughout most profiles (PAN-005, representative profile BO0614).

SOIL FERTILITY: The soils of the lower floodplains are moderately fertile, but those on the higher areas are little better than the higher peneplain areas of the region.

Topsoil Analyses:

Terrain Component BO-0002.1, Mean soil analytical data BO0612:pH, ECEC medium; Ca, Mg high; K, Na low; P medium.

Terrain Component BO-0002.2, Mean soil analytical data BO0614: 			 		pH, ECE,  Ca, K, Na and P low; Mg medium; Al high.

VEGETATION: Tropical semi-evergreen seasonal forests with emergent heights of 15-35m cover the higher floodplains.  Brazil nut trees and rubber trees are present on the better drained soils.  On the lower parts swamp forests predominate and are inundated anually.

FORESTRY PRODUCTION: Besides Brazil nut and rubber collection, there is some selective cutting of valuable hardwoods, particularly mahogany.

AGRICULTURAL AND ANIMAL PRODUCTION: On the higher well drained parts, there are occasional vegetable and fruit gardens; however, there always exists the danger of sudden floods.

SOTER UNIT:  BO-0008

LOCATION:	This poorly drained plains unit is located south of the Madre de Dios river between the Manupare and Beni rivers, and also between the Beni and Yata rivers (see photo 4).

AREA:	50500 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 2 Terrain Components BO-0008.1 (85%) and BO-0008.2 (15%), which represent poorly drained and imperfectly drained areas respectively.

ALTITUDE:	160 - 170 m, median altitude 165 m; lower in the Yata region.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary origin. (SC3Q).

HYDROLOGY: Due to its slightly concave land surface and the occasional presence of impermeable plinthite soil layers, drainage is slow.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils on the poorly drained surfaces, often have a plinthite layer within 40 cm of the surface with loamy, dark brown topsoils over clayey, dark grey subsoils.  (PAN-006, representative profile BO0036).

The soils on the imperfectly drained parts have clay textured, dark brown topsoils over light yellowish brown, gleyed subsoils.  (PAN-007, representative profile BO0004).

SOIL FERTILITY: The fertility of the soils is poor, Al levels are often high.

Topsoil Analyses:

Terrain Component BO-0008.1, Mean soil analytical data BO0036: pH, ECEC medium; Ca, Mg, K and Na low; P high; Al medium.

Terrain Component BO-0008.2, Mean soil analytical data BO0004: pH, ECEC, Ca, Mg, K, Na and P low; Al high.

VEGETATION: The forest height and composition changes with the soil drainage. An evergreen shrub type of forest with a average emergent height of 10m is found on the shallowest soils.  A palm dominated forest with an average emergent height of 20m, is found on less marginal soils.  A tropical semi-evergreen seasonal forest with rubber trees and an average emergent height of 30 m, is found on the better drained soils.

FORESTRY PRODUCTION: There is little activity in this unit; only some rubber and palm fruits are collected.

AGRICULTURAL AND ANIMAL PRODUCTION:  Only some minor shifting agriculture.

SOTER UNIT:  BO-0010

LOCATION:	The unit is found to the west of the Beni river, south of the Madre de Dios river.

AREA:	340740 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This uneven plains unit has been subdivided into 3 Terrain Components BO-0010.1 (50%), BO-0010.2 (40%) and BO-0010.3 (10%), which represent the better drained higher parts of the plains, slightly concave areas and stream line incisions respectively.

ALTITUDE:	135-165 m., median altitude 148 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary origin. (SC3Q).

HYDROLOGY: The most important streams which drain this unit are the San Rafael, the Genechiquia, the Canada and the Huipa streams; they are tributaries of the Beni and the Madre de Dios rivers.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The moderately well drained soils on the highest part of the landscape have clay textured, strong brown topsoils over yellowish red subsoils.  Small ironstone concretions are occasionally found throughout the profiles. (PAN-008, representative profile BO0197).

The imperfectly drained soils on the slightly concave areas of the plains have loamy, yellowish brown topsoils grading to clayey, yellowish red subsoils. Mottles and concretions are common in the subsoils. (PAN-009, representative profile BO0052).

The moderately well drained soils on the slopes of the incisions have loamy, yellowish brown topsoils and clayey, yellowish red subsoils with concretions.

(PAN-010, representative profile BO0618).

SOIL FERTILITY: These soils are poorly supplied with plant nutrients; Al levels are often high.

Topsoil Analyses:

Terrain Component BO-0010.1, Mean soil analytical data BO0197: pH medium; ECEC, Ca, Mg, K, Na and P low; Al medium.

Terrain Component BO-0010.2, Mean soil analytical data BO0052: pH, ECEC, Ca, Mg, K and Na low; P medium; Al medium.

Terrain Component BO-0010.3, Mean soil analytical data BO0618: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: The higher parts are covered by a moderately high, tropical semi-evergreen seasonal forest; emergent height 25-35 m, with a relatively thin underbush; Brazil nut trees are present.

The lower and dissected areas are covered by a low forest, height 7-20 m, with palms and patuju common.

FORESTRY PRODUCTION: There are many Brazil nut trees. Some valuable hard wood species, like mahogany and tumi may be found.

AGRICULTURAL AND ANIMAL PRODUCTION: The most important activity in this unit is the Brazil nut harvest.

SOTER UNIT:  BO-0011

LOCATION:	The unit is found in the southern savanna or "pampa" region of the study area, south of the Madre de Dios and Beni rivers (see photo 5).

AREA:	18044 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 2 Terrain Components, BO-0011.1 (75%) and BO-0011.2 (25%), which represent the flat and slightly lower parts and the more elevated parts of the landscape, respectively.  A distinct feature of this unit are the so-called "murundus", small mounds 30-100 cm high, and 50 to 300 cm wide, which support shrubs and occasional small trees.  Termite mounds are common.

ALTITUDE:	130-145 m; median altitude 135 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary origin (SC3Q).

HYDROLOGY: Due to underlying plinthite layers in the soils, a large proportion of the pampas have poor drainage conditions.  After a few days of rain, water-tables may reach the surface.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The poorly drained soils in the lower parts have loamy, dark grey topsoils overlying clayey light grey, gleyed subsoils.  (PAN-011, representative profile BO0648).

In the same Terrain Component (BO-0011.2), poorly drained soils 				are also found. (PAN-012, representative profile BO0649).

In many profiles, mottles and ironstone (Fe-Mn) concretions are seen. Plinthite layers can often be found at depths ranging from 10-150 cm.  Laterite concretions with size and abundance ranging from 2-30 cm. and 5-80% respectively, are found on the soil surface in some areas.

SOIL FERTILITY: The inherent fertility of these soils is low, Al levels are often high.

Topsoil Analyses:

Terrain Component BO-0011.1, Mean soil analytical data BO0648: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0011.2, Mean soil analytical data BO0649: pH medium; ECEC, Ca, Mg, K and Na low; P medium; Al medium.

VEGETATION: Shrubs and occasional small trees are found scattered throughout the savannas, generally on the "murundus" or small mounds.  The pampas are burnt annually.

AGRICULTURAL AND ANIMAL PRODUCTION: Only extensive cattle grazing is practised.

SOTER UNIT:  BO-0014

LOCATION:	The unit is located in the eastern part of the Pando Department, between the Orthon and the Madre de Dios river.

AREA:	43652 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit can be subdivided into 3 Terrain Components, BO-0014.1 (60%), BO-0014.2 (35%) and BO-0014.3 (5%), which represent the high, flat well drained part of the peneplain, the high, flat imperfectly drained sectors and the stream line incisions respectively.  A few communities are located on the river sides.

ALTITUDE:	130-150 m; median altitude 140 m.

LITHOLOGY:   Siltstones, mudstones, claystones of the Holocene Period (SC3H).

HYDROLOGY:  This narrow unit is drained to both the north, the Orthon river, and the south, the Madre de Dios river.  The water level in the streams follows the water level of the two rivers.  Within a week, levels can change several meters, so that canoes are necessary to cross normally fordable streams.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils on the higher, better drained parts of the peneplain have loamy, yellowish-brown topsoils grading to yellowish-red subsoils.  They are well drained, and few concretions are present. (PAN-013, representative profile BO0023).

The soils on the higher imperfectly drained surfaces have loamy textures and yellowish brown topsoils over brownish yellow subsoils.  Due to the imperfect drainage conditions, many mottles and concretions are seen in the subsoils. (PAN-014, representative profile BO0018).

The soils on the sloping sites of the incisions have loamy yellowish brown topsoils, changing to clayey, strong-brown subsoils.  Generally these soils are moderately well drained; few mottles and concretions can be found in the subsoils. (PAN-010,  like Terrain Component BO-0010.31, with representative profile BO0618.)

SOIL FERTILITY: The nutrient levels in these soils are low; Al levels tend to be high.

Topsoil Analyses:

Terrain Component BO-0014.1, Mean soil analytical data BO0023: pH, ECEC, Ca, Mg medium; K, Na low; P high; Al medium.

Terrain Component BO-0014.2, Mean soil analytical data BO0018: pH, ECEC, Ca, K and Na low; Mg, P medium; Al high.

Terrain Component BO-0014.3, Mean soil analytical data BO0618: pH, ECEC, Ca, Mg, K, Na low; P medium; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest, emergents 25-35 m, which contains a considerable amount of rubber and Brazil nut trees.  The underbush is relatively thin.  Less than 5% has been cut-over.

FORESTRY PRODUCTION: Rubber and Brazil nut collection.

AGRICULTURAL AND ANIMAL PRODUCTION: Small-scale vegetable and fruit gardens on the better drained soils.

SOTER UNIT:  BO-0015

LOCATION:	The unit is located in the eastern part of the Pando Department, between the Orthon and the Madre de Dios river.

AREA:	16484 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 3 Terrain Components, BO-0015.1 (70%), BO-0015.2 (25%) and BO-0015.3 (5%), which represent the high, flat well drained surfaces of the peneplain, the intermediate flat, moderately drained portions and stream line incisions respectively.

ALTITUDE:	128-135 m; median altitude 130 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Holocene Period (SC3H).

HYDROLOGY: This unit drains to both the north, the Orthon river and to the south, the Madre de Dios river.  The water level in its streams follows the water level of the two rivers; within a week, levels can rise several meters.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils on the higher, well drained parts have loamy, yellowish-brown topsoils over clayey, yellowish-red subsoils.  Few concretions are present throughout the profiles. (PAN-015, representative profile BO0025).

The soils on the slighty lower, moderately drained parts have lighter textures, only grading to clays in the lower horizons.  Concretions maybe seen in the subsoils.

(PAN-016, representative profile BO0022).

The soils in the incisions follow a sequence.  Generally they are moderately well drained on the slopes and poorly drained near the "valley" floors.  Few mottles and concretions are present in the subsoils. (PAN-010,  like Terrain Component BO-0010.31, with representative profile BO0618.)

SOIL FERTILITY: The nutrients levels in those soils are moderate to low.

Topsoil Analyses:

Terrain Component BO-0015.1, Mean soil analytical data BO0025: pH, ECEC, Ca, Mg medium; K, Na low; P high; Al medium.

Terrain Component BO-0015.2, Mean soil analytical data BO0022: pH low; ECEC, Ca, Mg and P medium; K, Na low; Al medium.

Terrain Component BO-0015.3, Mean soil analytical data BO0618: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest, average emergent height 25-35 m, covers this unit. Rubber trees are plentiful along the streams, Brazil nut trees are more common on the higher parts of the peneplain. In the eastern part of this unit close to Riberalta, up to 50% of the lands are cut over; fruit and vegetable gardens are kept to supply that city.

FORESTRY PRODUCTION: Rubber and Brazil nut collection.

AGRICULTURAL AND ANIMAL PRODUCTION: Small-scale fruit and vegetable gardens on the better drained soils.

SOTER UNIT:  BO-0016

LOCATION:	The Orthon and Tahuamanu river floodplains, which cross the study area from west to east (see photo 6).

AREA:	314740 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This floodplains unit has been subdivided into 2 Terrain Components, BO-0016.1 (65%) and BO-0016.2 (20%), which represent the higher, better  drained terraces, and the lower drained parts respectively.  The permanent water surface is estimated at 15% of the area.

ALTITUDE:	130-200 m; median altitude 145 m.

LITHOLOGY:    Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: Many small streams drain into this river system.  The most important is the Navenda stream, a tributary of the Tahuamana river.  The rivers are navigable by small boats, but are dangerous in the dry season due to half hidden tree trunks.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils on the higher, imperfect drained floodplain terraces have loamy textures with silty clay subsoils and light brownish grey colours. (PAN-017, representative profile BO0684).

The soils on the poorly drained terraces have silty clay textures with pale brown 			topsoils over light brownish-grey subsoils.  Mottles are seen throughout most                   		profiles. (PAN-018, representative profile BO0683).

SOIL FERTILITY: Fertility is moderate to low in these soils.

Topsoil Analyses:

Terrain Component BO-0016.1, Mean soil analytical data BO0684: pH, ECEC medium; Ca, Mg high; K, Na low; P medium.

Terrain Component BO-0016.2, Mean soil analytical data BO0683: pH, ECEC, Ca, K, Na and P low; Mg medium; Al high.

VEGETATION: A tropical swamp forest.  Rubber trees are common.

FORESTRY PRODUCTION: Rubber and palm fruits, including asaí collection.

AGRICULTURAL AND ANIMAL PRODUCTION: On the higher, well drained terraces vegetables and fruits are occasionally cultivated.  There exists the danger of sudden floods.

SOTER UNIT:  BO-0017

LOCATION:	The unit is found in the center northern part of the Pando Department.

AREA:	399620 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Components, BO-0017.1(90%) and BO-0017.2 (10%), which represent the flat, high part of the peneplain, and its dissected portions respectively.

ALTITUDE:	150-170 m; median altitude 165 m.

LITHOLOGY: Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: Apart from the Negro river, which drains the central part, the Mamo, Carnaval and Todos Santos streams also provide drainage for the area.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the flat high areas of the peneplain have loamy, yellowish-red topsoils and clayey, red subsoils.  Very few concretions are found in the subsoil. (PAN-019, representative profile BO0008).  In the same Terrain Component, moderately well drained soils which have loamy, strong brown topsoils and clayey, yellowish-red subsoils, are found.  Few concretions are present in the lower horizons. (PAN-020, representative profile BO0010).

The moderately well drained soils on the dissected parts have loamy, yellowish-brown topsoils grading to clayey, yellowish-red subsoils.  Few concretions are found in the subsoil. (PAN-021, representative profile BO0021).

SOIL FERTILITY: The soil analyses show that these soils are very low in major plant nutrients and have high Al levels.

Topsoil Analyses:

Terrain Component BO-0017.1, Mean soil analytical data BO0008: pH low; ECEC, Ca, Mg and P medium; K, Na low; Al medium.

Terrain Component BO-0017.1, Mean soil analytical data BO0010: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0017.2, Mean soil analytical data BO0021: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: The unit is covered with a high tropical semi-evergreen seasonal forest, emergent height 25-40m. Brazil nut and rubber trees are common.  The under-bush contains palms and jatata on the more poorly drained soils.

FORESTRY PRODUCTION: The forest contains a substantial Brazil nut and rubber tree population together with valuable hardwood species.  The latter are being felled at an accelerating rate.

AGRICULTURAL AND ANIMAL PRODUCTION: The most important activity in this unit is Brazil nut harvesting.

SOTER UNIT:  BO-0018

LOCATION:	The unit is located along the Orthon and the Pacahuara rivers, north-west of Riberalta.

AREA:	275244 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 3 Terrain Components, BO-0018.1(75%), BO-0018.2 (20%) and BO-0018.3 (5%), which represent the intermediate flat surfaces, the higher flat parts of the peneplain and the stream line incisions respectively.

ALTITUDE:	The altitude ranges from 140 m in the north, to 160 m in the south; median altitude 145 m.

LITHOLOGY: Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The unit is drained to the north by the Mamo, the Pacahuara and the Negro rivers, and to the south by the Orthon and the Madre de Dios rivers.  The lowest parts have drainage problems, and ground water can almost reach the surface.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:	The soils on the intermediate surfaces are imperfectly drained, and have loamy textures, yellowish brown topsoils and brownish yellow subsoils. Many mottles and concretions can be found in the subsoil. (PAN-022, representative profile BO0018).

The well drained soils on the higher surfaces of the landscape have loamy dark-red topsoils and clayey, red subsoils. (PAN-023, representative profile BO0032).

The well drained soils of the incisions slopes have a loamy, brown topsoils and clayey, yellowish-red subsoils, with concretions. (PAN-024, representative profile BO0170).

SOIL FERTILITY: These soils are poorly supplied with plant nutrients and have high Al levels.

Topsoil Analyses:

Terrain Component BO-0018.1, Mean soil analytical data BO0018: pH, ECEC, Ca, K and Na low; Mg and P medium; Al high.

Terrain Component BO-0018.2, Mean soil analytical data BO0032: pH, ECEC, Ca, K and Na low; Mg medium; P high; Al medium.

Terrain Component BO-0018.3, Mean soil analytical data BO0170: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: Where the drainage is impeded, patuju is common in the understory and the tropical semi-evergreen seasonal forest tends to be lower (20-30 m) and more open; Rubber trees are common.  On the higher, well drained soils almost no patuju is present, and both rubber and Brazil nut trees are plentiful; emergent are 30-40 m. high.  On the sloping sites the emergent height is about 25-35 m, and both rubber and Brazil nut trees are present in considerable amounts.  The underbush is thin and contains many small palms (3 m).

FORESTRY PRODUCTION: Rubber trees are quite common in this unit; a latex processing plant was constructed at Ingavi in 1992.

AGRICULTURAL AND ANIMAL PRODUCTION: Some small cut-over areas are cultivated for food gardens; the most important activity is rubber tapping.

SOTER UNIT:  BO-0019

LOCATION:	The unit is found in the eastern part of the Pando department, north of Riberalta.

AREA:	84312 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 4 Terrain Components BO-0019.1 (50%), BO-0019.2 (42%), BO-0019.3 (8%), which represent the slightly undulating part, the highest part and the stream line incisions.

ALTITUDE:	118-122 m; median altitude 120 m.

LITHOLOGY: Siltstones, mudstones, claystones; Holocene Period (SC3H).

HYDROLOGY: Among others the Santa Rosa stream drains the area; it is a tributary of the Negro river.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June to September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The moderately well drained soils on the slightly undulating parts of the peneplain have yellowish brown coloured, loamy  or clay topsoils over brown clayey subsoils. Small ironstone concretions are present throughout the soil profiles. (PAN-025, representative profile BO0005).

The well drained soils on the flat, higher parts of the peneplain have clay textures, strong brown coloured topsoils and yellowish-red subsoils. Few concretions are present in the subsoil. (PAN-026, representative profile BO0007).

The moderately well drained soils of the incision slopes have loamy, yellowish-brown topsoil grading to clayey, strong-brown subsoils.  Mottles and few concretions are found occasionally in the subsoils. (PAN-027, representative profile BO0635).

SOIL FERTILITY: The analytical data indicates that these are low fertility soils with high Al levels.

Topsoil Analyses:

Terrain Component BO-0019.1, Mean soil analytical data BO0005: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0019.2, Mean soil analytical data BO0007: pH, Ca, Mg, K and Na low; ECEC and P medium; Al high.

Terrain Component BO-0019.3, Mean soil analytical data BO0635: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest; emergents 25-40 m, containing many Brazil nut and rubber trees.  Patuju is common in the under-brush of the imperfectly drained soils.

FORESTRY PRODUCTION: The Brazil nut tree population of the unit is high.  The quantity of valuable hardwood species is rapidly diminishing due to selective felling.

AGRICULTURAL AND ANIMAL PRODUCTION:  The most important activity is the harvest of Brazil nuts and rubber collection.  Some shifting cultivation.

SOTER UNIT:  BO-0020

LOCATION:	The unit is found in the north-eastern part of the region encompassing the mouth of the Beni river, near the village of Cachuela Esperanza.

AREA:	88020 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 3 Terrain Components BO-0020.1 (75%), BO-0020.2 (20%) BO-0020.3 (5%), which represent the flat intermediate parts, the more elevated parts and the incisions respectively.  Rock out-crops of the Brazilian shield are clearly visible.

ALTITUDE:	100-140 m; median altitude 120 m.

LITHOLOGY:  Siltstones, mudstones, claystones of Quaternary origin with minor outcrops of Precambrian granites, gneiss and schists (SC3QP).

HYDROLOGY: The unit is dissected by the Beni river.  The Mamoré river drains the eastern part.

CLIMATE:	Guayaramerin:  Mean annual rainfall about 1450 mm, mean annual temperature 26.6°C.  There are 5 climatically dry months, May until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The moderately well drained soils on the flat, intermediate surfaces are clay textured, with yellowish brown topsoils over yellowish red subsoils. Mottles and Fe-Mn concretions can be found in the subsoil. (PAN-028, representative profile BO0180).

The soils on the more elevated parts are moderately well drained, and have yellowish-brown coloured clay loam topsoils and clayey subsoils.  Many concretions are present throughout the soil profile. (PAN-029, representative profile BO0179).

The poorly drained soils of the incision floors have greyish brown colours, loamy topsoils and clay subsoils. Mottles and concretions are present throughout the profile. (PAN-030, representative profile BO0638).

SOIL FERTILITY: The fertility of these soils is generally poor; Al levels high.

Topsoil Analyses:

Terrain Component BO-0020.1, Mean soil analytical data BO0180: pH, ECEC, Ca, Mg, K, Na and P low; Al high.

Terrain Component BO-0020.2, Mean soil analytical data BO0179: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0020.3, Mean soil analytical data BO0638: pH low; ECEC, Ca, Mg medium; K and Na low; Al high.

VEGETATION: Tropical semi-evergreen seasonal forest of median height covers this unit, with Brazil nut trees common on well drained soils.  In areas with impeded drainage patuju and palms can be found.  Less than 5% is cut over.

FORESTRY PRODUCTION: Brazil nut and rubber collection.

AGRICULTURAL AND ANIMAL PRODUCTION: Some shifting cultivation.

SOTER UNIT:  BO-0021

LOCATION:	The unit is found in the north eastern part of the Pando department.

AREA:	176132 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 2 Terrain Components BO-0021.1 (70%) and BO-0021.2 (30%), which represent the flat, higher areas and the stream incisions respectively.

ALTITUDE:	115-125 m; median altitude 122 m.

LITHOLOGY:    Siltstones, mudstones, claystones of the Quaternary Period (SC3Q).

HYDROLOGY: The Negro river drains the central part, and the Pacahuara river the nothern part; both are tributaries of the Abuna river.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The moderately well drained soils on the higher flat parts of the landscape have yellowish brown colours, clay topsoils and subsoils.  Concretions are common in these soils. (PAN-031, representative profile BO0634).

The imperfectly drained soils of the lower parts of the incisions have clay loam, yellowish brown topsoils over clayey, yellowish-red subsoils, which are mottled and contain concretions throughout the profile. (PAN-032, representative profile BO0175).

SOIL FERTILITY: The fertility of these soils is poor; Al levels are generally high.

Topsoil Analyses:

Terrain Component BO-0021.1, Mean soil analytical data BO0634: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0021.2, Mean soil analytical data BO0175: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: A medium height, tropical semi-evergreen seasonal forest, 25-35 m. covers the higher parts.  On the lower parts the underbush is thicker, and contains patuju.  Brazil nut and rubber trees are present.

FORESTRY PRODUCTION: Brazil nut and Rubber collection.

AGRICULTURAL AND ANIMAL PRODUCTION:  The most important activity is the harvest of Brazil nuts and rubber tapping. Some shifting cultivation for food gardens.

SOTER UNIT:  BO-0025

LOCATION:	This unit, the Beni river and its floodplains, cuts across the region from the south to the north-east.

AREA:	62796 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: The unit has been subdivided into 2 Terrain Components, BO-0025.1 (65%) and BO-0025.2 (10%), which represent the lower poorly drained parts and the higher better drained terraces respectively.  The permanent water surface amounts to 25%, the floodplain is 4 to 12 km. wide.

ALTITUDE:	130-155 m; median altitude 140 m.

LITHOLOGY:    Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: Close to Riberalta the Madre de Dios river joins the Beni river, and at the Brazilian frontier it joins Mamoré river, giving rise to the Madera river.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:	The soils on the poorly drained parts have silty clay textures, grey topsoils and light brownish grey, gleyed subsoils.  Mottles can be found throughout the profile. (PAN-033, representative profile BO0208).

The soils on the higher better drained floodplain terraces have loam textures, yellowish brown topsoils and yellowish red subsoils. (PAN-034, representative profile BO0654).

SOIL FERTILITY: The fertility of these soils is moderate to poor.

Topsoil Analyses:

Terrain Component BO-0025.1, Mean soil analytical data BO0208: pH, ECEC medium; Ca, Mg high; K and Na low; P medium.

Terrain Component BO-0025.2, Mean soil analitical data BO0654: pH, ECEC, Ca, K, Na and P low; Mg medium; Al high.

VEGETATION: Tropical swamp forest 7-15m high covers most of this unit, with the presence of Ambaibo Nigra, palms and rubber trees on the lower parts, and an occasional Brazil nut tree on the high better drained parts.

FORESTRY PRODUCTION: Rubber collection and some selective felling.  Considerable extensions of Ambaibo Negra.

AGRICULTURAL AND ANIMAL PRODUCTION: An occasional small fruit and vegetable garden on the high parts.  Some dry season extensive cattle ranching.

SOTER UNIT:  BO-0026

LOCATION:	The unit is found in the north eastern part of the Pando department, on the eastern side of the Negro river, and south of the Abuna river (see photo 7).

AREA:	252424 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 3 Terrain Components BO-0026.1 (65%), BO-0026.2 (30%) and BO-0026.3 (5%), which represent the flat higher surfaces, the slightly concave intermediate parts of such and the incisions respectively.

ALTITUDE:	105-115 m; median altitude 110 m.

LITHOLOGY:    Siltstones, mudstones and claystones of the Holocene period (SC3H).

HYDROLOGY: Tributaries of the Negro river, the Cachuelita, the Campo Lindo and the Tamboiqui streams drain the unit.

CLIMATE:	Guayaramerin:  Mean annual rainfall about 1450 mm, mean annual temperature 26.6°C.  There are 5 climatically dry months, May until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerlies.

SOILS:		The moderate to well drained soils on the high and flat surfaces have clay brown topsoils and clayey yellowish-red and red subsoils, which contain Fe-Mn concretions throughout the profile. (PAN-035, representative profile BO0184).

The soils on the concave intermediate areas of the plains have clay textures and yellowish brown topsoils over yellowish red subsoils; drainage is generally moderate and impeded. Concretions and mottles are found in the subsoil. (PAN-036, representative profile BO0181).

The impeded drained soils of the lower parts of the incisions, have loamy light brownish grey topsoils over clayey light grey gleyed subsoils with mottles and concretions. (PAN-037, representative profile BO0638).

SOIL FERTILITY: The fertility of these soils is poor, Al levels are high.

Topsoil Analyses:

Terrain Component BO-0026.1, Mean soil analytical data BO0184: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0026.2, Mean soil analytical data BO0181: pH, ECEC, Ca, Mg, K, Na and P low; Al high.

Terrain Component BO-0026.3, Mean soil analytical data BO0638: pH low; ECEC, Ca, Mg medium; K and Na low; Al high.

VEGETATION: A low tropical semi-evergreen seasonal forest, emergent height 15-30m with Brazil nut trees, covers much of the unit.  The underbush contains many small palms and patuju on soils with impeded drainage.

FORESTRY PRODUCTION: This forest is only sparsely populated with rubber and Brazil nut trees

AGRICULTURAL AND ANIMAL PRODUCTION: The soils are poor and only a little shifting agriculture is practised.

SOTER UNIT:  BO-0027

LOCATION:	The unit is found in the north-eastern corner of the Pando department.

AREA:	74284 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit is subdivided into 3 Terrain Components BO-0027.1 (35%), BO-0027.2 (45%) and BO-0027.3 (20%), which represent the flat higher surfaces, the slightly concave intermediate parts and the lower parts respectively.

ALTITUDE:	95-103 m; median altitude 100 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Holocene period (SC3H).

HYDROLOGY: To the north and to the east, this unit drains to the Abuna and the Madera river respectively.

CLIMATE:	Guayaramerin:  Mean annual rainfall about 1450 mm, mean annual temperature 26.6°C.  There are 5 climatically dry months, May until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils in the Terrain Component BO-27.1 are caracterised by impeded drainage conditions.  The imperfectly drained soils have yellowish brown topsoils over clay, brown subsoils, mottles and concretions are found throughout the profiles (PAN-038, representative profile BO0667)

The imperfectly drained, gleyed soils in Terrain Component BO-0027.2 				have yellowish brown colours and clay textures with mottles and 				concretions (PAN-039, representative profile BO0666.

The slowly drained soils in Terrain Component BO-0027.3 have yellowish 			brown colours and loamy textures (PAN-40, representative profile 				BO0191).

SOIL FERTILITY: The natural fertility of these soils is low.  Al levels tend to be high.

Topsoil Analyses:

Terrain Component BO-0027.1, Mean soil analytical data BO0667: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0027.2, Mean soil analytical data BO0666: pH, ECEC, Ca, Mg, K, Na and P low; Al medium.

Terrain Component BO-0027.3, Mean soil analytical data BO0191: 				pH, ECEC, Ca, Mg, K, Na and P low; Al medium.

VEGETATION:  A low hydromorphic forest with a preminance of palms and patuju.

FORESTRY PRODUCTION: Limited to minor extraction.

AGRICULTURAL AND ANIMAL PRODUCTION: Owing to its inherently poorly drained conditions, there is virtually no agriculture practised in this unit.

SOTER UNIT:  BO-0028

LOCATION:	The unit is found in the eastern part of the Pando Department, between the Pacahuara and the Negro river

(see photo 8).

AREA:	161244 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 2 Terrain Components, BO-0028.11 (65%) and BO-0028.21 (35%), which represent a slighty elevated flat plain surface and the dissected areas, respectively.

ALTITUDE:	130-140 m; median altitude 135 m.

LITHOLOGY:   Siltstones, mudstones, claystones; Quaternary period (SC3Q).

HYDROLOGY: The unit is dissected by many small streams which drain to the Pacahuara river in the north, and to the Negro river in the south.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:	        The soils on the slighty elevated plain surface are moderately well drained and have clay textures yellowish red topsoil colours and red subsoils with occasional small concretions. (PAN-041, representative profile BO0628).

The imperfectly drained soil in the lower dissected areas have clay loam, yellowish brown topsoils and clay yellowish-red subsoils with mottles and concretions. (PAN-042, representative profile BO0629).

SOIL FERTILITY: The fertility of these soils is low, as evidenced from the analytical data.  Al level are high.

Topsoil Analyses:

Terrain Component BO-0028.1, Mean soil analytical data BO0628: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0028.2, Mean soil analytical data BO0629: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

VEGETATION: On the sloping and lower parts, the tropical semi-evergreen seasonal forest is poorly developed, locally called "monte ralo".  It is an open forest and the underbush mainly consists of palms and patuju in the poorly drained areas.  Only occasional Brazil nut trees are present; rubber trees are more frequent.  On the more elevated parts, the soil is better drained, which results in a better developed tropical semi-evergreen seasonal forest with more Brazil nut trees.

FORESTRY PRODUCTION: This area used is a rich rubber area, but following the decline of the rubber prices many of the communities were abandoned, and the people moved away mainly to Riberalta.  Still small amounts of Brazil nuts are extracted.

AGRICULTURAL AND ANIMAL PRODUCTION: Only occasional shifting cultivation due to poor soil conditions.

SOTER UNIT:  BO-0029

LOCATION:	The unit is found in the north-eastern part of the Pando department, between the Abuna and the Pacahuara rivers.

AREA:	205332 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This plains unit has been subdivided into 3 Terrain Components, BO-0029.1 (65%), BO-0029.2 (35%) and BO-0029.3 (5%), which represent most of the flat surfaces, flat slightly higher areas and stream incisions respectively.

ALTITUDE:	120-130 m; median altitude 125 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Holocene period (SC3H).

HYDROLOGY: The most important streams which drain this unit are the Illa, the Buen Futuro and the Grande Atea, all tributaries of the Abuna river.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The imperfectly drained soils on most of the flat surfaces have loam textures with yellowish brown topsoils over clay subsoils.  Mottles and occasionally small Fe-Mn concretions are present throughout the profile. (PAN-043, representative profile BO0664).

The moderately well drained soils on the higher parts have clay textures, yellowish-red and red coloured topsoils and red subsoils. (PAN-041, like Terrain Component BO-0028.11, with representative profile BO0628.)

The poorly drained soils in the incisions have clay textures, greyish-brown topsoils and grey, gleyed subsoils.  Many mottles are seen throughout the profiles.(PAN-005, like Terrain Component BO-0002.21, with representative profile BO0614.)

SOIL FERTILITY: The fertility of these soils is poor.  Al levels are often high.

Topsoil Analyses:

Terrain Component BO-0029.1, Mean soil analytical data BO0664: pH, ECEC, Ca, Mg, K and Na low; Mg and P medium; Al high.

Terrain Component BO-0029.2, Mean soil analytical data BO0628: pH, ECEC, Ca, Mg, K and Na low; P medium; Al high.

Terrain Component BO-0029.3, MEan soil analytical data BO0614: pH, ECEC medium; Ca and Mg high; K and Na low; P medium.

VEGETATION: A low tropical semi-evergreen seasonal forest 15-30 m, covers the unit; only a few Brazil nut trees with few and small "cocos", but a considerable amount rubber trees are found.

FORESTRY PRODUCTION: Significant amounts of rubber are found in this unit.

AGRICULTURAL AND ANIMAL PRODUCTION: Only very minor areas of shifting agriculture is practised.

SOTER UNIT:  BO-0030

LOCATION:	The Abuna river and its tributary the Manu river, with their floodplains along the frontier between Brazil and northern Bolivia (see photo 9).

AREA:	75000 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This unit is summarized by one Terrain Component, BO-0030.1, to represent the floodplains.  The permanent water surface of the unit amounts to 10%.  The floodplains are 1 to 6 km. wide, and 20-40% of such consist of higher well drained terraces.

ALTITUDE:	100-160 m; median altitude 130 m.

LITHOLOGY:     Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: The Abuna river is a tributary of the Madera river. In some places, e.g. Montevideo, it has a narrow floodplain and a deeply incised river bed which results in water level rises of more than 12 m during the wet season.  The latter starts in December and lasts until April.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The poorly drained soils on the lower parts of the floodplains have silty clay, light grey, gleyed topsoils and subsoils. Mottles and occassionally small Fe-Mn concretions are seen throughout the profiles. (PAN-044,  like Tearrain Component BO-0031.11, with representative profile BO0633.)

The imperfectly drained soils on the highest floodplain terraces, which are 			seasonally flooded, have silty clay light brownish grey topsoils and silty 				clay light grey subsoils. Many mottles are seen and small Fe-Mn 				concretions can sometimes be found in the subsoils.

(PAN-045, like Terrain Component BO-0031.21,with representative profile 			BO0659.)

SOIL FERTILITY: The nutrient supplying capacity of these soils is marginal to moderate.

Topsoil Analyses:

Terrain Component BO-0030.1, Mean soil analytical data BO0633: pH low; ECEC, Ca and Mg medium; K, Na low; Al medium.

Terrain Component BO-0030.1, Mean soil analytical data BO0659: pH low; ECEC, Ca and Mg medium; K, Na low; Al high.

VEGETATION: A tropical annually flooded forest covers the floodplain.  On the well drained terraces the average emergent height is 25-35 m, but very few Brazil nut trees are present.  On the poorly drained parts the forest is lower 15-25 m; palms and patuju are abundant. Rubber trees are common throughout the unit.

FORESTRY PRODUCTION: Rubber and palm fruits collection.

AGRICULTURAL AND ANIMAL PRODUCTION: During the dry season just a few fruits and vegetables are produced on the highest parts, but there is always the risk of a sudden flood.

SOTER UNIT:  BO-0031

LOCATION:	The unit consists of the Negro and the Pacahuara river floodplains (see photo 10).

AREA:	28140 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This floodplain unit is summarized by one Terrain Component, BO-0031.1.  Its permanent water surface covers 10%.  The floodplains are less than 2 km. wide; 20% of such consist of higher, imperfectly drained terraces.

ALTITUDE:	110-130 m; median altitude 120 m.

LITHOLOGY:    Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: The rivers are tributaries of the Abuna river.  Every year during the wet season which starts in December and last 5 months, the flood plains are completely inundated.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The poorly drained soil on the lower parts of the flood plain, BO-0036, have silt, light brownish-grey gleyed topsoils and subsoils.  Many mottles can be found throughout these profiles. (PAN-044, representative profile BO0633).

The imperfectly drained soils on the highest floodplain terraces, which are 			seasonally flooded, have silty claybrown topsoils and silty clay brownish-			yellow subsoils. Many mottles are seen and many small Fe-Mn 					concretions can sometimes be found in the subsoils. (PAN-045, 				representative profile BO0661).

SOIL FERTILITY: The fertility of these soils is poor to moderate.

Topsoil Analyses:

Terrain Component BO-0031.1, Mean soil analytical data BO0633: pH low; ECEC, Ca and Mg medium; K, Na low; Al medium.

Terrain Component BO-0031.1, Mean soil analytical data BO0661: pH low; ECEC, Ca and Mg medium; K, Na low; Al high.

VEGETATION: A tropical seasonally flooded forest covers the unit. On the well drained terraces emergent height is 25-35 m, but very few Brazil nut tree are present.  On the poorly drained parts the forest is lower, 15-25 m; palms and patuju are abundant, rubber trees common.

FORESTRY PRODUCTION: Rubber and palm fruit collection.

AGRICULTURAL AND ANIMAL PRODUCTION: Very little potential.

SOTER UNIT:  BO-0032

LOCATION:	The unit is found in the north-eastern part of the Pando department, south of the Abuna river.

AREA:	204592 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplane has been subdivided into 3 Terrain Components, BO-0032.1 (55%), BO-0032.2 (35%) and BO-0032.3 (10%), which represent the better drained flat surface, dissected areas and the poorer drained surfaces, respectively.

ALTITUDE:	125-147 m; median altitude 145 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary period (SC3Q).

HYDROLOGY:  Apart from the Manú river, this unit is drained by the Trahia and the Santa Rosa streams.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The moderately well drained soils on the almost flat surfaces have loamy strong brown topsoils over clayey brown subsoils.  Very few concretions are found in the subsoils. (PAN-046, representative profile BO0668).

The soils of the dissected areas are moderately well drained and have loamy to clay yellowish-brown topsoils over clayey strong brown soils which sometimes contain small ironstone concretions. (PAN-018, like Terrain Component BO-0018.1, with representative profile BO0018.)

The imperfectly drained soils on the concave plain surfaces have clay loam textures, with yellowish brown topsoils over clayey pale brown subsoils.  Mottles and small ironstones concretions are present throughout the profiles. (PAN-050, representative profile BO0135).

SOIL FERTILITY: Analitically the soil in this unit have low pHs, fertility is low.  Al levels are high.

Topsoil Analyses:

Terrain Component BO-0032.1, Mean soil analytical data BO0668: pH low; ECEC, Ca, Mg and P medium; K, Na low; Al medium.

Terrain Component BO-0032.2, Mean soil analytical data BO0018: pH low; ECEC, Ca, Mg and P medium; K, Na low; Al high.

Terrain Component BO-0032.3, Mean soil analytical data BO0135: pH, ECEC, Ca, K, Na low; Mg, P medium; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest, emergent height 20-30 m, with a considerable amount of rubber and Brazil nut trees, covers the unit.  The underbush contains patuju on the more poorly drained soils.

FORESTRY PRODUCTION: Rubber and Brazil nut collection.

AGRICULTURAL AND ANIMAL PRODUCTION: A small amount of shifting cultivation.

SOTER UNIT:  BO-0033

LOCATION:	The unit is found in the central-northern part of the Pando department, north of the Orthon river.

AREA:	154200 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Componente, BO-0033.1 (90%) and BO-0033.2 (10%) which represent the flatish plain surfaces and stream line incisions respectively.

ALTITUDE:	160-170 m; median altitude 167 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY:  Various streams drain this unit, the most important are the Rin, the Dos de Oro and the Carichon streams, all tributaries of the Abuna river.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the flat plains surfaces have loamy textures, yellowish red topsoils and red subsoils with clay textures, which sometimes contain a few small ironstone Fe-Mn concretions. (PAN-047, representative profile BO0013).

The well drained soils on the incision slopes have loam textures and strong brown coloured topsoils over red subsoils. Small ironstone concretions and sometimes a few reddish mottles maybe found throughout the profile.

(PAN-048, representative profile BO0011).

SOIL FERTILITY: The fertility of these soils is low.  Al levels tend to be high.

Topsoil Analyses:

Terrain Component BO-0033.1, Mean soil analytical data BO0047: pH, ECEC, Ca, K, Na low; Mg, P medium; Al medium.

Terrain Component BO-0033.2, Mean soil analytical data BO0011: pH low; ECEC, Ca and P medium; Mg, K and Na low; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest, emergent height 25-40 m, with many Brazil nut and rubber trees, covers the unit.  The underbush is thin and easy to walk through.

FORESTRY PRODUCTION: Apart from Brazil nuts and rubber this forest contains valuable hardwood species which are being felled at an accelerating rate.

AGRICULTURAL AND ANIMAL PRODUCTION: Most of the cut-over land is used for cattle ranching; only a small percentage for shifting agriculture.

SOTER UNIT:  BO-0034

LOCATION:	The unit is found in the north-western part of the Pando department, between the Chipamanu and the Tahuamanu rivers.

AREA:	326912 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This strongly dissected peneplain unit has been subdivided into 2 Terrain Componente, BO-0034.1 (45%) and BO-0034.2 (55%), which represent the dissected areas and the residual flat surfaces respectively.

ALTITUDE:	160-270 m; median altitude 200 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: Many streams dissect the unit, the most important being the Chipamanu, the Kharamanu and the Timbay streams.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The moderately well drained soils of the dissected sloping terrain have loamy brown topsoils over more clayey yellowish red subsoils. Many Fe-Mn concretions and a few mottles are found throughout the profiles.

(PAN-049, representative profile BO0136).

The soils on the residual plain surfaces have loamy texture, brown topsoils over red clay subsoils.  Particularly on the poorly drained phases of these soils mottles and a few small ironstone concretions are found throughout the profiles. PAN-050, representative profile BO0135 represent the predominantly well drained soils.

SOIL FERTILITY: The fertility of these soils is low; Al levels are high.

Topsoil Analyses:

Terrain Component BO-0034.1, Mean soil analytical data BO0136: pH low; ECEC, Ca, Mg and P medium; K, Na low; Al high.

Terrain Component BO-0034.2, Mean soil analytical data BO0135: pH, ECEC, Ca, K, Na low; Mg and P medium; Al medium.

VEGETATION: A tropical semi-evergreen seasonal forest with emergents of variable heights, 15-35 m, due to the unit's topography, covers the unit.  Brazil nut and rubber trees are common.

FORESTRY PRODUCTION: Brazil nut and rubber collection.

AGRICULTURAL AND ANIMAL PRODUCTION: Next to small scale farming, the most important activity is the harvest of Brazil nuts and latex extraction.

SOTER UNIT:  BO-0035

LOCATION:	The unit is found in the central-northern part of the Pando department, immediately south of the Brazilian border.

AREA:	128372 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This hilly unit has been subdivided into 2 Terrain Components, BO-0035.1 (90%) and BO-0035.2 (10%) which represent the hills and residual flat, high peneplain surfaces respectively.

ALTITUDE:	150-161 m; median altitude 155 m.

LITHOLOGY:   Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The Abuná and Rapirran rivers, and the El Rin and the Muymanu streams drain the unit.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the hill slopes often have loamy light yellowish brown topsoils over clayey yellowish-red subsoils.  Small ironstone concretions are common and a few mottles maybe seen throughout the profiles.

(PAN-051, representative profile BO0154).

The well drained soils on the high residual peneplain surfaces have loamy, brown topsoils over clayey, yellowish-red to red subsoils. (PAN-052, representative profile BO0156).

SOIL FERTILITY: The fertility of these soils is poor; Al levels are high.

Topsoil Analyses:

Terrain Component BO-0035.1, Mean soil analytical data BO0154: pH low; ECEC, Ca, Mg and P medium; K, Na low; Al high.

Terrain Component BO-0035.2, Mean soil analytical data BO0156: pH, ECEC, Ca, K, Na and P low; Mg medium; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest with emergents at variable heights (15-35m), covers the unit.  Next to Brazil nut and rubber trees, mahogamy, tumi and cedro are found. On some sloping parts, a scrub type of forest with abundant bamboo can be found.

FORESTRY PRODUCTION: Brazil nut and rubber collection, and selective felling of valuable hardwoods.

AGRICULTURAL AND ANIMAL PRODUCTION: Small-scale agriculture, yuca, plátanos, cacao, and some pasture establishment for cattle ranching.

SOTER UNIT:  BO-0036

LOCATION:	The unit is found in the north-western part of the Pando department, along the northern side of the Tahuamanu river (see photo 11).

AREA:	215292 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Components, BO-0036.1 (65%) and BO-0036.2 (35%) which represent the flat high surfaces and the dissected areas respectively.

ALTITUDE:	200-287 m; median altitude 265 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: Apart from the Garape Preto stream, the unit is drained by many small tributaries of the Chipamanu and the Tahuamanu rivers.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS: 	The well drained soils on the flat, high surfaces have loam to clay reddish-brown topsoils and clayey, red subsoils.  A few, very small ironstone concretions can sometimes be found throughout the profiles. (PAN-053, representative profile BO0080).

The well drained soils of the sloping dissected areas have loamy, brown topsoils and clayey yellowish red subsoils.  A few concretions are occasionally found throughout the profiles. (PAN-054, representative profile BO0104).

SOIL FERTILITY: The fertility of these soils is poor; Al levels are often high.

Topsoil Analyses:

Terrain Component BO-0036.1, Mean soil analytical data BO0080: pH, ECEC, Ca, K, Na and P low; Mg medium; Al high.

Terrain Component BO-0036.2, Mean soil analytical data BO0104: pH, ECEC, K, Na, Mg and P low; Ca medium; Al high.

VEGETATION: A high tropical semi-evergreen seasonal forest; 25-40 m, with many Brazil nut and rubber trees covers this unit.

FORESTRY PRODUCTION: Brazil nut and rubber collection, and the selective felling of valuable hardwood species.  The latter is occuring at an alarming rate.

AGRICULTURAL AND ANIMAL PRODUCTION:  Along the roads, the forest has been cleared to establish pasture for cattle ranching.  Some fruits and vegetables for the Cobija market are also produced, on a shifting cultivation basis.

SOTER UNIT:  BO-0037

LOCATION:	This unit is found in the north-western part of the Pando department (see photo 12).

AREA:	181064 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This hill unit has been subdivided into 2 Terrain Componente, BO-0037.1 (90%) and BO-0037.2 (10%), which represent the hills and the high, flat residual peneplain surfaces respectively.

ALTITUDE:	200-286 m; median altitude 250 m.

LITHOLOGY:     Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY:  Apart from the Navenda and Garape Preto streams, many little tributaries of the Acre river drain the unit.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the hill slopes have a loamy yellowish-brown topsoils over clayey yellowish-red subsoils which may have a few small ironstone concretions. (PAN-055, representative profile BO0165).

The well drained soils on the residual peneplain surfaces have a loam, reddish brown topsoils over clayey red subsoils. Concretions and mottles can sometimes be found throughout the profiles. (PAN-056, representative profile BO0148).

SOIL FERTILITY: Fertility is low and Al levels are high in these soils.

Topsoil Analyses:

Terrain Component BO-0037.1, Mean soil analytical data BO0165: pH, Ca, K and Na low; ECEC and Mg medium; Al high.

Terrain Component BO-0037.2, Mean soil analytical data BO0148: pH, ECEC, Ca, K, Na and P low; Mg medium; Al high.

VEGETATION: A high tropical semi-evergreen seasonal forest, 25-45 m, covers the unit.  It has a thin underbush.  Rubber and Brazil nut trees are common.

FORESTRY PRODUCTION: Brazil nut and rubber collection. Selective felling of valuable hardwood species is increasing rapidly.

AGRICULTURAL AND ANIMAL PRODUCTION: Along the roads the forest has been cleared to establesh pasture for cattle grazing.  Some fruits and vegetables are produced for the Cobija market.

SOTER UNIT:  BO-0038

LOCATION:	The unit is found in the north-western most part of the Pando department, along the Brzilian and Peruvian borders (see photo 13).

AREA:	116152 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This hill unit contains two Terrain Components, BO-0038.1 (60%) and BO-0038.2 (40%) which represent its undulating, dissected topography and the high flat surfaces of the peneplain respectively.

ALTITUDE:	200-290 m; median altitude 250 m.

LITHOLOGY:   Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The most important streams which drain the region are the Navenda, the San Miguel, the Piapi and the Henohaya streams; the latter three are tributaries of the Acre river.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils in this unit are well drained and have loamy brown topsoils over clayey, yellowish-red subsoils which may have mottles and small ironstone concretions. (PAN-057, representative profile BO0146).

The soils in Terrain Component B0-0038.2 are moderate drained and 					have loamy textures, yellowish brown topsoil and red clay subsoil. These 				soils are situated in the  higher parts and have some mottles in the profile.                 			(PAN-058, representative profile BO0674)

SOIL FERTILITY:  The analitical data indicates that these soils are poorly supplied with plant nutrients.  Al levels are high.

Topsoil Analyses:

Terrain Component BO-0038.1, Mean soil analytical data BO0146: pH, Ca, K, Na low; ECEC, Mg medium; P medium; Al high.

Terrain Component BO-0038.2, Mean soil analytical data 					BO0674, pH, Ca, K,	Na, ECEC and P low; Mg  medium; Al high.

VEGETATION: A medium to high tropical semi-evergreen seasonal forest, 15-30m, covers the unit with a small proportion of trees more than 35 m. high.  The forest contains Brazil nut, rubber and mahogany trees.  The underbush is dense, and in some parts Bamboo is abundant.

FORESTRY PRODUCTION: Brazil nut and rubber collection together with felling of valuable hardwood species.  Few valuable trees are left in the vicinity of roads.

AGRICULTURAL AND ANIMAL PRODUCTION: The main activity is cattle ranching.  A few fruits and vegetables are produced for the Cobija market.

SOTER UNIT:  BO-0039

LOCATION:	The unit is found in the western extremity of the Pando department, between the Tahuamanu and the Manuripi rivers.

AREA:	321980 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Components, BO-0039.1 (80%) and BO-0039.2 (20%), which represent the high, flat surfaces of the peneplain and its dissected areas respectively.  The depth of minor valleys can exceed 30 m.

ALTITUDE:	190-285 m; median altitude 250 m.

LITHOLOGY:   Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The central part of the unit is drained by the Buyuyo stream which is a tributary of the Manuripi river.  To the north, the region drains for the Tahumanu river, and to the south, the Manuripi river.

CLIMATE:	Cobija: Mean annual precipitation about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the high, flat surfaces have loam to clay, yellowish-red topsoils over clayey red subsoils which may have a few small ironstone concretions. (PAN-059, representative profile BO0145).

The well drained soils in the lower dissected areas have loamy yellowish-			red topsoils and red clay subsoils, sometimes concretions are found in 				some profiles  (PAN-060, representative profile BO0097).

SOIL FERTILITY: Analytically the soils in this unit have medium pHs, and low fertility conditions.

Topsoil Analyses:

Terrain Component BO-0039.1, Mean soil analytical data BO0145: pH, Ca, Mg, Na medium; ECEC, K low; Al medium.

Terrain Component BO-0039.2, Mean soil analytical data BO0097: pH, ECEC, Mg, K and Na low; Ca medium; Al medium.

VEGETATION: A tropical semi-evergreen seasonal forest with emergents of 35-45 m, covers the unit. The underbush is thin; the trees trunks can reach diameters of 2 meters.  Occasionally, a tree is more than 50 meters high. Rubber, Brazil nuts and native cacao trees are common.  Many palms were obsurved on the  imperfectly drained soils. Less than 2% has been cut down for grazing land.

FORESTRY PRODUCTION: Rubber and Brazil nut extraction.

AGRICULTURAL AND ANIMAL PRODUCTION: Some shifting cultivation for local food supplies.

SOTER UNIT:  BO-0040

LOCATION:	The unit is found in the centre of the Pando Department between the Orthon and the Madre de Dios river.

AREA:	108552 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Components, BO-0040.1 (95%) and BO-0040.2 (5%), which represent the flat peneplain surfaces and the stream line incisions respectively.

ALTITUDE:	150-185 m; median altitude 165 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary period (SC3Q).

HYDROLOGY: The Negro, Mati and the Diasada streams, tributaries of the Madre de Dios river, and the Porvenir, Esperanza and San Miguel streams, tributaries of the Orthon river, drain the unit.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the flat, high surfaces have loamy textures, with yellowish-red topsoils over red clay subsoils.  The latter may have a few small ironstone concretions.(PAN-061, representative profile BO0082).

The well drained soils on the sloping terrain have loam textures, and strong brown topsoils over red subsoils.  Small ironstone concretions are common, and sometimes a few mottles can be seen throughout the profiles.

(PAN-048, like Terrain Component BO-0033.21, with representative 				profile BO0011.)

SOIL FERTILITY: The fertility of these soils is generally poor; Al levels are often high.

Topsoil Analyses:

Terrain Component BO-0040.1, Mean soil analytical data BO0082: pH, ECEC, Ca, K and Na low; Mg, P medium; Al medium.

Terrain Component BO-0040.2, Mean soil analytical data BO0011: pH low; ECEC, Ca and P medium; Mg, K and Na low; Al high.

VEGETATION: A high, tropical semi-evergreen seasonal forest, emergent height 30-40 m, with many Brazil nut and rubber trees, covers the unit.

FORESTRY PRODUCTION: Brazil nut and rubber collection, together with selective felling of valuable timber species, including mahogany and cedro.

AGRICULTURAL AND ANIMAL PRODUCTION: Some shifting agriculture for local food supplies.

SOTER UNIT:  BO-0041

LOCATION:	The unit is found between the Tahuamanu and the Manuripi rivers in the central-western part of the region.

AREA:	144092 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Components, BO-0041.1 (80%) and BO-0041.2 (20%), which represent the flat, high peneplain surfaces and the dissected parts respectively.

ALTITUDE:	165-257 m; median altitude 200 m.

LITHOLOGY:     Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The Buyuyo stream drains the western sector of this unit; the eastern sector is drained by various small streams.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the high, flat surfaces have loam, yellowish red topsoils over clayey red subsoils which may have a few small ironstone concretions.

(PAN-062, representative profile BO0090).

The well drained soils of the dissected areas have similar properties to the soils on the high, flat surfaces. (PAN-060, like Terrain Component BO-0039.21, with representative profile BO0097.)

SOIL FERTILITY: The fertility of these soils is poor to moderate.

Topsoil Analyses:

Terrain Component BO-0041.1, Mean soil analytical data  BO0090: pH, Ca, Mg, Na medium; ECEC, K low; Al medium.

Terrain Component BO-0041.2, Mean soil analytical data BO0097: pH, ECEC, Mg, K and Na low; Ca medium; Al medium.

VEGETATION: A high tropical semi-evergreen seasonal forest, 35-45 m, with tree trunk diameters of 1.5-2.0 m, covers the unit, Brazil nut and rubber trees are common.  The underbush is thin.

FORESTRY PRODUCTION: Brazil nut and rubber collection, together with the selective felling of valuable hardwood species.  Until now, the latter has been done only on a small scale.

AGRICULTURAL AND ANIMAL PRODUCTION: Small areas of shifting cultivation for the production of yuca, rice, and plátanos.  Minor areas cleared for cattle ranching.

SOTER UNIT:  BO-0042

LOCATION:	The unit is found in the south-western most part of the Pando department, next to the Peruvian Border, and along the Madre de Dios river, north of Chivé.

AREA:	369568 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided in 3 Terrain Components, BO-0042.1 (85%), BO-0042.2 (10%) and BO-0042.3 (5%), which represent the flat well drained surfaces, the slighty concave imperfectly drained surfaces, and the dissected areas respectively.

ALTITUDE:	200-230 m; median altitude 225 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: To the north, the unit drains to the Manuripi river; in the south to the Madre de Dios river.  Streams, with wide 1-2 km. floodplains dissect the unit.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months May to August, during the cool season when cold southern fronts "surazos", alternate with northerly winds.

SOILS:		The well drained soils on the high, flat surfaces have loam-clay yellowish-red topsoils over clayey, red subsoils. (PAN-063, representative profile BO0140).

The moderately well drained soils on the high surfaces (slightly concave areas), with clay and loamy textures and yellowish red topsoils over red clay subsoils.(PAN-064, representative profile BO0044).

The dissected, sloping areas have imperfect drained soils loam-clay 				textures and may have some very fine ironstone concretions.  The 				topsoils are brown and the subsoils yellowish-red.

(PAN-065, representative profile BO0047).

SOIL FERTILITY: The fertility of these soils is poor; Al levels tend to be high.

Topsoil Analyses:

Terrain Component BO-0042.1, Mean soil analytical data BO0140: pH low; ECEC, Ca, Mg medium; K, Na and P low; Al high.

Terrain Component BO-0042.2, Mean soil analytical data BO0044: pH low; ECEC, Ca and Mg medium; K and Na low; Al medium.

Terrain Component BO-0042.3, Mean soil analytical data BO0047: pH low; ECEC, Mg medium; Ca, K and Na low; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest with average emergent height 35-45 m, covers this unit, which is certainly one of the most beautiful forests of this region.  Brazil nut trees with trunks of more than 2 m. diameter, and 50 meters high, are quite common.  The underbush is relatively thin.  Forest height diminishes when drainage conditions are impeded; more patuju and palms are present.  Brazil nut and rubber trees are found in all three Terrain Components.  Less than 5% has been cut-over.

FORESTRY PRODUCTION: Brazil nuts and rubber extraction, and forest enrichment with valuable hardwood species, e.g. mahogany.

AGRICULTURAL AND ANIMAL PRODUCTION: Small scale vegetable and fruit gardens.

SOTER UNIT:  BO-0043

LOCATION:	The unit is found in the central part of the region between the Madre de Dios and the Manuripi rivers (see photo 14).

AREA:	129836 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplane unit has been subdivided into 2 Terrain Components, BO-0043.1 (90%) and BO-0043.2 (10%) which represent the high, flat surfaces and the stream line respectively.

ALTITUDE:	190-230 m; median altitude 210 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY:  Tributaries of the Manuripi and the Madre de Dios rivers drain the area.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:	        The well drained soils on the higher, flat surfaces have clay textures with brown topsoils over red subsoils; the latter may contain a few ironstone concretions. (PAN-066, representative profile BO0038).

The well drained soils on the slopes of the incisions have clay textures and pale brown to yellow-red topsoils over red subsoils. (PAN-067, representative profile BO0037).

SOIL FERTILITY: The fertility of these soils is poor; Al levels are medium to high.

Topsoil Analyses:

Terrain Component BO-0043.1, Mean soil analytical data BO0038: pH, ECEC, Ca, K, Na low; Mg, P medium; Al medium.

Terrain Component BO-0043.2, Mean soil analytical data BO0037: pH low; ECEC, Ca and P medium; Mg, K and Na low; Al high.

VEGETATION: A high tropical semi-evergreen seasonal forest, 25-45 m, with many Brazil nut trees and some rubber trees, covers the unit.  The underbush is thin and easy to walk through.  Valuable hardwood species are present.

FORESTRY PRODUCTION: Brazil nut and rubber collection, together with selective felling of valuable hardwood species.  The latter is proceding at an alarming rate.

AGRICULTURAL AND ANIMAL PRODUCTION: Some small scale, shifting agriculture (yuca, rice, maize),

SOTER UNIT:  BO-0044

LOCATION:	The unit, the Manuripi river its tributaries, and its floodplains, is found in the western part of the Pando department (see photo 15).

AREA:	198416 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This floodplain unit has been subdivided into 2 Terrain Components, BO-0044.1 (60%) and BO-0044.2 (25%), which represent the lower poorly drained parts and the higher, better drained terraces respectively.  The permanent water surface amounts to 15%.  The floodplain ranges from 4 to 15 km. wide.

ALTITUDE:	160-190 m; median altitude 180 m.

LITHOLOGY:    Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: The Buyuyo and the Florida streams are the most important tributaries of the Manuripi river.  Close to Puerto Rico the Tahuamanu river is navigable but in the dry season many half hidden trunks makes it very dangerous for larger vessels.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils on the poorly drained parts, have silty clay textures, dark greyish brown coloured topsoils and greyish brown subsoils.  Many mottles can be seen throughout most profiles. (PAN-068, representative profile BO0689)

The soil on the higher imperfectly drained floodplain terraces also have silty clay textures and greyish brown gleyed horizons. Many mottles and concretions can be found throughout the profiles.(PAN-069, representative profile BO0677).

SOIL FERTILITY: The fertility of these soils is moderate.

Topsoil Analyses:

Terrain Component BO-0044.1, Mean soil analytical data BO0689: pH, ECEC medium; Ca, Mg, K and Na low; P medium.

Terrain Component BO-0044.2, Mean soil analytical data BO0677: pH, ECEC, Ca, MG, K, P medium; Na low; Al high.

VEGETATION: A tropical floodplain forest, 7-15 m hig with Ambaibo Nigra, and palms, on the poorly drained soils.  On the better drained soils, a higher forest, 15-30 m, in which rubber trees are common.

FORESTRY PRODUCTION: Rubber and just a little Brazil nut collection, together with some palm fruits.

AGRICULTURAL AND ANIMAL PRODUCTION: An occasional small fruit and vegetable garden is found on the highest terraces but, there always exists the danger of sudden floods.

SOTER UNIT:  BO-0045

LOCATION:	The unit is found between the Tahuamanu and the Manuripi rivers, south of Cobija.

AREA:	38644 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 3 Terrain Components, BO-0045.1 (55%), BO-0045.2 (25%) and BO-0045.3 (20%), which represent the flat, slightly concave imperfectly drained surfaces, the well drained flat surfaces, and the dissected areas respectively.

ALTITUDE:	190-270 m; median altitude 220 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The unit is drained by two small rivers, the Supaymanu and the Buyuyo, which are both tributaries of the Manuripi river.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained clayey soils on the flat, high surfaces have yellowish-red topsoils and generally red subsoils. (PAN-070, representative profile BO0091).

The imperfectly drained soils on the slightly concave surfaces have loamy 			grading to clay textures.  Topsoil and the subsoil colours are strong 				brown.  Mottles can be seen in the subsoils. (PAN-071, representative 				profile BO0701).

The well drained soils on the dissected sloping parts have brown topsoils and a yellowish red subsoil.  well drained flat parts, but topsoils tend to be loamy and subsoils clayey. (PAN-072, representative profile BO0700).

SOIL FERTILITY: These soils are poorly supplied with the major plant nutrients.  Al levels are high.

Topsoil Analyses:

Terrain Component BO-0045.1, Mean soil analytical data BO0091: pH low; ECEC, Mg medium; Ca, K and Na low; Al high.

Terrain Component BO-0045.2, Mean soil analytical data BO0701: pH, ECEC, Mg, K, Na and P low; Ca medium.

Terrain Component BO-0045.3, Mean soil analytical data BO0700: pH, ECEC, Mg, K and Na low; Ca medium; Al medium.

VEGETATION: A tropical semi-evergreen seasonal forest with emergent heights of 30-40 m. Brazil nut and rubber trees are common. Palms and Patuju are common on the imperfectly drained soils.  Less than 3% has been cut- over.

FORESTRY PRODUCTION: Rubber and Brazil nut collection, with selective feeling of hardwoods.

AGRICULTURAL AND ANIMAL PRODUCTION: Some shifting cultivation.

SOTER UNIT:  BO-0046

LOCATION:	This unit is found between the Madre de Dios and the Manuripi rivers in the south-western part of the study area (see photo 16).

AREA:	171472 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: The peneplain unit has been subdivided into 3 Terrain Components, BO-0046.1 (75%), BO-0046.2 (20%) and BO-0046.3 (5%), which represent the high, flat surfaces, the dissected stream line areas and the slightly concave portions of the high, flat surfaces respectively.

ALTITUDE:	165-195 m; median altitude 185 m.

LITHOLOGY:    Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The unit is drained by tributaries of the Manurimi river.  Some minor streams drain forwards the Madre de Dios river.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months from May until August, during the cool season when cold southern fronts "surazos", alternate with warm northerly winds.

SOILS:		The soils on the high, flat, well drained surfaces have a loam to clay yellowish-red topsoils and clayey, red subsoils. (PAN-073, representative profile BO0041).

The loam to clay soils of the sloping dissected areas are well drained, with strong brown coloured topsoils grading to yellowish-red subsoils. Occasionally small ironstone concretions are found in their subsoils. (PAN-074, representative profile BO0646).

The clayey soils of the slightly concave parts of the high surfaces are imperfectly well drained, with strong brown topsoils grading to yellowish-red, mottled subsoils containing a few small ironstone concretions. (PAN-075, representative profile BO0077).

SOIL FERTILITY: The analytical data indicates that these soils are poorly supplied with plant nutrient.  Al levels are high.

Topsoil Analyses:

Terrain Component BO-0046.1 Mean soil analytical data BO0041: pH, ECEC, Ca, Mg, K and P low; Al high.

Terrain Component BO-0046.2, Mean soil analytical data BO0646: pH, Mg and P medium; ECEC, K, Na low; Al medium.

Terrain Component BO-0046.3, Mean soil analytical data BO0077: pH, ECEC, Ca, Na low; Mg, P and K medium; ECEC low; Al high

VEGETATION: Tropical semi-evergreen seasonal forest, with emergent heights of 25-45 m.  Brazil nut and rubber trees are common.

FORESTRY PRODUCTION: This unit is rich in rubber and Brazil nut trees.  Mahogany and cedro are found, but are being cut-out.

AGRICULTURAL AND ANIMAL PRODUCTION: Small areas of shifting cultivation, with yuca and plátanos.

SOTER UNIT:  BO-0047

LOCATION:	The unit is located in the southern-most part of the study area south of the Madre de Dios river and west of the Beni river, approximately 20 km. west of Fortaleza.

AREA:	138352 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided in 3 Terrain Components, BO-0047.1 (85%), BO-0047.2 (10%) and BO-0047.3 (5%), which represent the main slightly concave surface with impeded drainage, a better drained surface, and some shallow, 5 m. deep, stream incisions respectively.

ALTITUDE:	170-189 m; median altitude 185 m.

LITHOLOGY: Siltsones, mudstones, claystones; Quaternary period (SC3Q).

HYDROLOGY: Drainage from the unit is slow to both the Beni and Madre de Dios rivers.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months from May until August, during the cool season when cold southern fronts "surazos", alternate with warm northerly winds.

SOILS:		The slow to moderately slowly drained soils on the slightly concave surfaces, have yellowish-brown topsoils grading to pale brown subsoils.  Clay textures predominate.  Where the drainage is poor, soils are grayer; topsoil colours are light yellowish-brown and subsoils pale brown to grey and gleyed. Small ironstone concretions are found throughout the profiles. (PAN-076, representative profile BO0204).

The clayey soils of the moderately well drained flat surfaces have yellowish brown topsoils grading to strong brown subsoils.  Small ironstone concretions are plentiful un the subsoils. (PAN-077, representative profile BO0207).

SOIL FERTILITY: Fertility is low and Al levels high in those soils.

Topsoil Analyses:

Terrain Component BO-0047.1, Mean soil analytical data BO0204: pH low; ECEC, Mg medium; Ca, K, Na and P low; Al high.

Terrain Component BO-0047.2, Mean soil analytical data BO0207: pH, ECEC, Ca, Na and P low; Mg, K medium; Al medium.

VEGETATION: A tropical semi-evergreen seasonal forest hydrosere of medium height covers much of the unit. Brazil nut and rubber trees are common.  Less than 5% is cut-over, mostly confined to areas around "barracas" or collection centers.

FORESTRY PRODUCTION: The area around Fortaleza is well known for its high Brazil nut tree population.

AGRICULTURAL AND ANIMAL PRODUCTION: Small scale fruit and vegetable gardens.

SOTER UNIT:  BO-0048

LOCATION:	The unit is found in the south-western part of the study area, between the Manurimi and the Manupare rivers.

AREA:	109224 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 3 Terrain Components, BO-0048.1 (75%), BO-0048.2 (15%) and BO-0048.3 (10%), which represent the flat, high surfaces, the stream line incisions, and the slightly concave sectors of the plain surfaces, respectively.

ALTITUDE:	160-190 m; median altitude 185 m.

LITHOLOGY:    Siltsones, mudstones, claystones; Quaternary period (SC3Q).

HYDROLOGY: The San Manuel and the Zaramanu streams are the most important streams draining the area.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months, May until August, during the cool season when cold southern fronts "surazos", alternate with warm northerly winds.

SOILS:		The well drained soils on the flat, high surfaces have clay textures, yellowish-red topsoils and red subsoils. (PAN-078, representative profile BO0069).

The moderately drained soils in the lower parts of the incisions have loamy, strong-brown coloured topsoils and clayey red subsoils. (PAN_079, representative profile BO0034).

The moderately drained soils on the slightly concave sectors of the plain surfaces have clayey, yellowish- brown topsoils over clayey, yellowish red subsoil.

(PAN-080, representative profile BO0202).

SOIL FERTILITY: These soils are poorly supplied with most plant nutrients.  Al levels are high.

Topsoil Analyses:

Terrain Component BO-0048.1, Mean soil analytical data BO0069: pH, ECEC, Ca, Mg, K and P low; Al high.

Terrain Component BO-0048.2, Mean soil analytical data BO0034: pH, Mg and P medium; ECEC, Ca, K and Na low; Al medium.

Terrain Component BO-0048.3, Mean soil analytical data BO0202: pH, ECEC, Ca, Mg, K and Na low; P medium; Al medium.

VEGETATION: A high, tropical semi-evergreen seasonal forest, 30-45 m. with many Brazil nut and rubber trees.

FORESTRY PRODUCTION: Brazil nut and rubber collection, and some felling of hardwood species.

AGRICULTURAL AND ANIMAL PRODUCTION: Small areas of shifting cultivation, mainly for growing rice, maize, bananas and cassava.

SOTER UNIT:  BO-0049

LOCATION:	The unit is found along the eastern side of the Manupare river; 20 km. south of Sena.

AREA:	39696 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This dissected peneplain unit has been subdivided into 2 Terrain Components, BO-0049.1 (60%) and BO-0049.2 (40%), which represent the dissected sectors and high, residual flat surfaces respectively.

ALTITUDE:	155-170 m; median altitude 160 m.

LITHOLOGY:   Siltsones, mudstones, claystones; Quaternary period (SC3Q).

HYDROLOGY: The unit is closely dissected by little streams, coming from the higher lands the east and flowing to the Manupare river.

CLIMATE:	Riberalta: Mean annual rainfall about 1650 mm, mean annual temperature 26,9°C.  There are 4 dry months, June until September, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The sloping soils of the dissected sectors have a loam textures and strong-brown coloured topsoils over reddish-brown mottled subsoils.  Drainage varies according to site; some of the soils are well drained, other moderately slow.  The amount of mottles and presence of small ironstone concretions varies. (PAN-081, representative profile BO0035).

The clayey soils on the flat, imperfectly to well drained residual surfaces have strong brown coloured topsoils over yellowish red subsoils, with a few small ironstone concretions.  (PAN-082, representative profile BO0203).

SOIL FERTILITY: The inherent fertility of these soils is low.  Al levels one high.

Topsoil Analyses:

Terrain Component BO-0049.1, Mean soil analytical data BO0035: pH, Mg and P medium; ECEC, Ca, K and Na low; Al medium.

Terrain Component BO-0049.2, Mean soil analytical data BO0203: pH, ECEC, Ca and Na low; Mg, K and P medium; Al high.

VEGETATION: A tropical semi-evergreen seasonal forest of medium height 25-35 m, covers most of this unit.  Rubber trees and some Brazil nut trees are present.

FORESTRY PRODUCTION: Due to the low rubber and Brazil nut prices, some of the Barracas in this unit have been abandoned.  Transport costs are to high for this remote area.

AGRICULTURAL AND ANIMAL PRODUCTION: Some shifting agriculture.

SOTER UNIT:  BO-0050

LOCATION:	The unit is found in the south-western part of the study area, north of the Madre de Dios river.

AREA:	62684 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This unit consists of older, higher terraces of the Madre de Dios floodplains.  It has been subdivided into 3 Terrain Components, BO-0050.1 (60%), BO-0050.2 (35%) and BO-0050.3 (5%), which represent the moderate drained areas of the terraces, poorly drained areas and some shallow stream line incisions respectively.

ALTITUDE:	160-185 m; median altitude 180 m.

LITHOLOGY:   Older unconsolidated fluvial sediments (UFO).

HYDROLOGY: Minor streams drain this unit including the Chivé and the América streams, all tributaries of the Madre de Dios river.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months from May until August, with occasional cold southern fronts or "surazos".

SOILS:		The moderate drained soils of the terraces have loam textures and pale brown coloured topsoils over grey clay subsoils. (PAN-083, representative profile BO0624).

The poorly drained soils on the terraces have loamy  dark greyish topsoils over yellowish brown, mottled subsoils. (PAN-084, representative profile BO0078).

The poorly drained soils in the lower parts of the incisions near stream beds have silty textures, greyish-brown topsoils and a grey, often gleyed, subsoils.

(PAN-085, like Terrain Component BO-0025.11, with representative 				profile BO0208.)

SOIL FERTILITY: These soils are relatively well supplied with plant nutrients.

Topsoil Analyses:

Terrain Component BO-0050.1, Measn soil analytical data BO0624: pH low; ECEC, Ca, Mg and K medium; Na low; Al high.

Terrain Component BO-0050.2, Mean soil analytical data BO0078: pH, ECEC, Ca and Mg medium; K, Na and P low; Al medium.

Terrain Component BO-0050.3, Mean soil analytical data BO0208: pH, ECEC medium; Ca, Mg high; K, Na low; P medium.

VEGETATION: A low to medium tropical semi-evergreen seasonal forest, 20-30 m, with an occasional tree higher than 35 m, covers the unit.  The underbush is often dense. Rubber and Brazil nut trees may be found.

FORESTRY PRODUCTION: Besides Brazil nut and rubber collection, felling of valuable hardwood species including mahogany and tajibo.

AGRICULTURAL AND ANIMAL PRODUCTION: Tradicional products of the region, yuca, rice, maize and plátanos are cultivated on a very small scale near settlements.

SOTER UNIT:  BO-0052

LOCATION:	The unit comprizes the floodplains of the Sena, Manurimi and Manupare rivers, found approximately 160 km. SW of Riberalta.

AREA:	36680 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This unit is summarized by Terrain Component BO-0052.1.  Water covers about 20% of the unit in the dry season. The floodplains are 1-5 km. wide; 10% to 20% of such consist of higher, well drained terraces.

ALTITUDE:	160-175 m; median altitude 165 m.

LITHOLOGY:    Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: The Manurimi at the Manupare rivers join to form the Sena river, which is a tributary of the Madre de Dios.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months from May until August, during the cool season when cold southern fronts "surazos", alternate with warm northerly winds.

SOILS:		The poorly drained soils on the lower parts of the floodplain have silty clay pale brown topsoils and silty clay light brownish grey, gleyed subsoils.  Many mottles can be found throughout the profiles. (PAN-044, like Terrain Component BO-0031.11m, with represenative profile BO0633.)

The soils on the highest floodplain terraces, which are seasonally flooded, 			have silty clay, brownish grey topsoils over silty clay, light grey subsoils.  			Many mottles and small ironstone concretions can sometimes be found in 			their subsoils. (PAN-086, like Terrain Component BO-0031.21, with 				representative profile BO0659.)

SOIL FERTILITY: These soils are moderately fertile.

Topsoil Analyses:

Terrain Component BO-0052.1, Mean soil analytical data BO0633: pH low; ECEC, Ca and Mg medium; K, Na low; Al medium.

Terrain Component BO-0052.1, Mean soil analytical data BO0659: pH low; ECEC, Ca and Mg medium; K, Na low; Al high.

VEGETATION: A tropical semi-evergreen, seasonally flooded, low to medium height forest, 7-25 m, with bamboo, patuju, and some rubber trees, covers most of the unit.

FORESTRY PRODUCTION: Collection of palm fruits and some rubber.

AGRICULTURAL AND ANIMAL PRODUCTION: Virtually unused.

SOTER UNIT:  BO-0053

LOCATION:	The unit, the Acre river and its floodplains, forms the frontier between Brazil and Bolivia from Bolpebra to Cobija.

AREA:	8924 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This unit is summarized as Terrain Component, BO-0053.1, to represent the flooded alluvial terraces. The permanent water surface covers 15% of the area.  The floodplains are 0.5 to 4 km. wide; 20-40% of which consists of higher terraces.

ALTITUDE:	175-185 m; median altitude 180 m.

LITHOLOGY:   Recent unconsolidated fluvial sediments (UFR).

HYDROLOGY: In the beginning of this century a large ship from Europe sailed up to Cobija; this is impossible these days, although the river is deeply incised in the surrounding landform.  Every year flooding starts in December and last 5 months, covering most of the unit.

CLIMATE:	Cobija: Mean annual rainfall about 1730 mm, mean annual temperature 24.7°C.  There are 3 dry months, June, July and August, during the cool season when cold southerly fronts "surazos", alternate with warm northerly winds.

SOILS:		The poorly drained soils on the lower terraces have loamy, dark brown topsoilsand silty clay, light grey, gleyed subsoils.  Many mottles can be found  throughout these profiles. (PAN-044, representative profile BO0699)

The imperfect drained soils on the highest terraces, which are seasonally 			flooded, have sandy loam,dark greyish topsoils over loamy, light yellowish 			brown,gleyed subsoils. Mottles can be found in the subsoils. (PAN-087, 				representative profile BO0676).

SOIL FERTILITY: These soils have moderate to low fertility levels.

Topsoil Analyses:

Terrain Component BO-0053.1, Mean soil analytical data BO0699: pH low; ECEC, Ca and Mg medium; K, Na low; Al medium.

Terrain Component BO-0053.1, Mean soil analytical data BO0676: pH low; ECEC, Ca and Mg medium; K, Na low; Al high.

VEGETATION: A tropical, semi-evergreen, seasonally flooded forest covers the unit. On the well drained terraces, the average emergent height is 25-35 m. Very few Brazil nut trees are present. On the poorly drained parts the forest is lower, 15-30 m, and palms and patuju are abundant.  Rubber trees are common throughout the unit.

FORESTRY PRODUCTION: Rubber and palm fruit collection.

AGRICULTURAL AND ANIMAL PRODUCTION: During the dry season a few fruits and vegetables are produced on the higher terraces.  There is always the risk of sudden floods.

SOTER UNIT:  BO-0054

LOCATION:	The unit is found in the south-western part of the Pando department near Peruvian border and the Madre de Dios river, 20 km. north of Chivé.

AREA:	8228 ha.

PHYSIOGRAPHY AND TOPOGRAPHY: This peneplain unit has been subdivided into 2 Terrain Components, BO-0054.1 (95%) and BO-0054.2 (5%), which represent a high, slightly concave surface and some shallow stream line incisions, respectively.

ALTITUDE:	175-180 m; median altitude 185 m.

LITHOLOGY:   Siltstones, mudstones, claystones; Quaternary with Tertiary influence (SC3QT).

HYDROLOGY: The unit has drainage problems; some parts are poorly drained others are better drained.  The Florida and the Tulapa streams drain this area.

CLIMATE:	Puerto Maldonado: Mean annual rainfall about 1925 mm, mean annual temperature 26.5°C.  There are 4 dry months from May until August, during the cool season when cold southern fronts "surazos", alternate with warm northerly winds.

SOILS:		The imperfectly drained soils on the high surfaces have clay textures and yellowish-brown topsoils grading to strong brown subsoils. Mottles and small ironstone concretions may be seen throughout the profiles.

(PAN-088, representative profile BO0045).

The well drained soils on the sloping sites have clay textures with yellowish-red topsoils overlying red subsoils. (PAN-064, representative profile BO0044).

SOIL FERTILITY: These soils are poorly supplied with plant nutrients.  Al levels are high.

Topsoil Analyses:

Terrain Component BO-0054.1, Mean soil analytical data BO0045: pH low; ECEC, Mg medium; Ca, K, Na and P low; Al high.

Terrain Component BO-0054.2, Mean soil analytical data BO0044: pH low; ECEC, Ca and Mg medium; K and Na low; Al medium.

VEGETATION: A tropical semi-evergreen seasonal forest hydrosere, of medium height (20-30 meters) covers the unit. Brazil nut and rubber trees are present.  Patuju and palms are common on the imperfectly drained soils.

FORESTRY PRODUCTION: Brazil nut and Rubber collection.

AGRICULTURAL AND ANIMAL PRODUCTION: Virtually no activity.

Photo 1: SOTER UNIT BO-0002.  Alluvial terrace of the  Madre de 		                 		      Dios river.

Photo 2:  SOTER UNIT BO-0002.  A hand made canal on alluvial terrace of the			                 Madre de Dios river.

Photo 3: SOTER UNIT BO-0002.  A poorly drained soil on an alluvial 				              terrace, near the Madre de Dios river.

Photo 4:  SOTER UNIT BO-0008.  Low forest on a soil with a plinthite 				               subhorizon layer, near Sena.

Photo 5: SOTER UNIT BO-0011.  Natural pampas in the south-east of the 			      	     Pando Department.

Photo 6:  SOTER UNIT BO-0016.  Poorly drained alluvial terrace,  					     Tahuamanu river.

Photo 7: SOTER UNIT BO-0026.  Vegetation on intermediate  part of 				                the peneplain.

Photo 8: SOTER UNIT BO-0028. Road for transport of Brazil nuts in the dry 				   season.

Photo 9: SOTER UNIT BO-0030.   Vegetation on alluvial terrace of the 				                Abuna river.

Photo 10: SOTER UNIT BO-0031.  Vegetation on alluvial terrace of the 			        	     Pacahuara river.

Photo11: SOTER UNIT BO-0036. Chaco after the first burning/clearing, near 			         Cobija.

Photo12: SOTER UNIT BO-0037.   Vegetation on the                              				peneplain with an undulating surface.

Photo13: SOTER UNIT BO-0038.  Vegetation on the                               				   dissected lands near Bolpebra.

Photo 14: SOTER UNIT BO-0043.  Intermediate part of  the 					         peneplain near the river Madre de Dios.

Photo 15: SOTER UNIT BO-0044. Vegetation on poorly                            				 drained alluvial plains, river Manuripi.

Photo 16: SOTER UNIT BO-0046.   Typical colours of the  higher 				         situated soils.

Photo 17: Brazilnuts, one of the natural products of the  Amazon forest.

Photo 18: Rubber (bolacha), one of the natural  products of 					        the Amazon forest.

SECTION  III

THE LAND USE ZONING MAP

The Land Use Zoning LUZ, map has been prepared as a provisional document to orient discussion on an approach to development in the Pando Department of the Bolivian Amazonia.  The zones demarcated indicate desirable land use categories that could lead to the sustained development of the region in an environmentally sound manner.  These zones take into account what is possible with regard to land use and forest management, in the light of their land resource characteristics and socio-economic circumstances.  Consequently, the objective of this section is to provide a brief summary of the several zones in terms of their potential with regard to land use, forest management and rural development, and to suggest what should be done to ensure that development will proceed on a sustained basis without destroying the unique tropical Amazonian forest environment of the department.

The Land Use Zones were defined and delineated as a result of interdisciplinary discussions following the completion of the inter-linked and separated studies carried out by the team members of both the soils studies and the vegetation studies of the department.

LAND USE ZONES

Eight land use zones were delineated following the team discussions, as shown on the Land Use Zoning map:

1.   SETTLEMENT AREAS

2.   EXTENSIVE RANCHING AREAS

3.   NATIVE FOREST RESERVES FOR FLORA

4.   NATIVE FOREST RESERVES FOR FAUNA

5.   EXTRACTIVE FOREST RESERVES

6.   MANAGED FOREST RESERVES

7.   CRITICAL WATERSHED REGIONS, WATER EROSION HAZARD

8.   CRITICAL WATERSHED REGIONS, FLOODING HAZARD

1. SETTLEMENT AREAS

The Land Resource Survey indicated that the soil resources of the department, although varied, are generally poor and have severe limitations for agricultural usage.  The better alluvial soils found along the flood plains of the rivers dissecting the region are almost invariably subject to severe flooding hazards.  Consequently, the choice of settlement areas for agricultural activities must be qualified from the outset by emphasizing that these have been delineated for non-intensive agricultural systems, and only as a response to the sociological realities of the recent migration movements within the region toward its capital city, Cobija.  In addition, an area of land was set aside for a group of indigenous peoples found in the study region itself.  The areas are not suitable for "colonization" by people from the over-populated highlands of Bolivia, as much better soil resources exist in other parts of the country.

Two settlement areas have been tentatively delineated:

a)  In the environs of Cobija.

b)  Along the western extremity of the Madre de Dios river in the vicinity of Chivé.

a)   Cobija

SOTER unit BO-0036, running from Cobija to Puerto Rico, an area of approximately 215,000ha, has been tentatively designated for more intensive settlement, although it is emphasized that this should not be encouraged due to its poor soil resources and relatively rich remaining forest resources.  The region envelopes the sector of the trans. Pandian highway running from Puerto Rico to Cobija.

This is a higher peneplain area with a median elevation of 265m, which is rapidly being cleared of its native forests for cattle ranching.  The soils of the region are inherently poor Ferralic (Oxisolic and Ultisolic) soils with medium to clay textures, that under native forest conditions are marginally more fertile than those found near Riberalta.  Topsoil pH averages about 5.0 and  exchangeable aluminum levels are about 1.0 cmol(+)/kg.  The regularly dissected nature of the landscape, coupled with its more intensive rainfall regime, imposes a serious soil erosion problem when the forest is removed, apart from the collateral soil chemical and internal physical degradation.

The original forest covering the region was a tropical semi-evergreen seasonal forest, with fewer dry season deciduous emergents than found in the forests near Riberalta.  In terms of non-timber products, it was, and still is in parts, a relatively rich source of both Brazil nuts and rubber, which are often seen growing virtually side by side in this region, along with native cacao. Over 43,000ha of this once majestic forest had already been cleared for cattle ranching by 1990, according to satellite imagery studies, refer to the PC Users Packet, Section ll, IDRISI files.  It is evident that forest clearing is accelerating, and that many more hectares of forest have been cleared since 1990!

Ranching following clearing has not been successful, and induced pastures are very degraded and unproductive.  Burnt Brazil nut trees standing alone in eroded pastures barely supporting one head of cattle per 5 to 8ha, is a typical current scenario.  In fact, one Brazil nut tree surviving in a pasture near Cobija, was found to have a diameter of 210cm at chest level, and although severely damaged by fire, was still producing nuts!

Forest removal in the region has already lead to the disappearance of several once viable native communities, which previously depended on the extraction of forest products.

Low intensity agro-forestry systems might be developed with the help of fertilizer and lime inputs.  However, settlers should be encouraged to leave a high proportion of their lands in forest, and an effort made to enhance the latter as a renewable source of both non-timber and timber products.  Over the considerable area of land that has already been cleared, improved pasture management and cattle raising practices may help slow down and eventually reverse the trend of soil degradation and decreasing production.   Nevertheless, further deforestation of these lands for cattle ranching should not be encouraged, and felling for timber carefully controlled.

b)  Chivé

An area of 91,000ha located along the Madre de Dios river near the Peruvian border has been zoned as a tribal region for the indigenous Toromonas Indian peoples, who have populated that region for many years.  It is delimited by SOTER unit BO-0050, which is an area of alluvial lands.  A large portion of these lands are susceptible to annual flooding.  They are covered by a medium to low tropical semi-evergreen forest, (a flood plain hydrosere), parts of which are relatively rich in both non-timber extractive products and timber including some of the last remaining mahogany stands along the Madre de Dios river.

Current logging for mahogany and other valuable timber species should be suspended pending the drawing up of territorial limits for the indigenous peoples of the zone.

2.  EXTENSIVE RANCHING

Minor areas of natural savanna lands encroach into the forested lands in and south-eastern quadrants of the region.  It is logical to zone these lands for extensive ranching, which is limited to SOTER unit BO-0011with predominately poorly drained lands covering some 18,000ha.  Apart from the predominance of drainage problems in these lands, they have acidic Planosolic soils that are very poorly supplied with plant nutrients.  Further, it is usual to find hardened plinthithic (iron-stone) surface gravels and subsurface horizons in these soils.

The native pasture on these lands support about one head of cattle per 7 to 10ha, although there is a little scope for improving the carrying capacity and general productivity of the savannas through the use of mineral supplements and the establishment of improved pastures in adjacent forested lands.  The clearing of adjacent forest lands for the establishment of supplementary pastures should not be unlimited, and possibly restricted to not exceed two hectares per 10ha of savannas.  Forests high in extractive products should be left alone.

3.  NATIVE FOREST RESERVES FOR FLORA

The area zoned as a native forest reserve for the conservation of native flora and its genetic diversity, covers an area with a very low rural population, and represents the major physiognomic forest types of the region.  These range from  low to tall semi-evergreen  seasonal to almost evergreen forests, along with their hydroseres.  It is envisaged that these forests can be used for the seasonal extraction of non-timber products, always provided that care is taken to maintain their floristic integrity.  In other words, settlement and timber extraction should be completely prohibited.  These forests are unique to western Amazonia, and should be preserved for future generations.

The area zoned consists of SOTER units BO-0039, BO-0042, BO-0043, BO-0044 and BO-0054.  These units have varying altitudes, land forms, lithology and soils, which is reflected as varying natural forests with considerable differences in physiognomy and floristic composition.  As relatively little is known about them, they might well prove a treasure throve in the years to come in terms of yet undiscovered plant species and genetic variability between species.  It is very interesting to see Brazil nut trees, rubber and native cacao growing in close proximity one to another on some of the soils of the several SOTER units, and it is believed that these remote forests are the center of origin of these and other species.  They should be declared a resource for humanity and protected as such.

4.   NATIVE FOREST RESERVES FOR FAUNA

Two areas have been chosen as forest reserves for fauna:

a)  An area in the north-eastern Pando department which is difficult to penetrate due to seasonal flooding or semi-flooding of a high proportion of its landscape, SOTER units BO-0027 and BO-0029 covering approximately 281,000ha and,

b)  An area of 115,000ha in the north-western corner of the Pando department which is hilly and would otherwise be categorized as a critical watershed region.

Area   a:

Area (a) in north-eastern Pando is virtually unpopulated by human beings, mainly because of its poorly drained soils.  From all accounts it has a high feline and reptilian populations together with a rich diversity of many other animal species, not only on the ground and in its streams but also living in or on its forest cover including monkeys and birds.  The area should also be treated as a floristic reserve, especially for palms and water tolerant species.

Area   b:

Area (b) in the N.W. corner of Pando has been zoned primarily as a non-human primate reserve, as it is the natural habitat of at least one rare species of monkey that could easily be lost if care is not taken for its preservation.  Further, many other animal and bird species are found in the area, which at the current rate of deforestation in the Cobija region and aggravated by local hunting habits, are likely to disappear from the region in the near future.

Soil-wise, the area is inherently poor, mainly nutrient-poor Ultisols, which would be subject to  a severe water erosion hazard should its forest cover be removed.  Consequently, it is axiomatic that no timber removal should take place in the region.  Further, non-timber extractive activities should also be prohibited due to the custom of local people to hunt while collecting forest products.

5.  EXTRACTIVE RESERVES.

Over 43% of the area, 2,745,000 ha, has been specifically zoned for extractive activities that should cause a minimum of damage to the forest ecosystem.   (Selective extractive activities, but under more closely managed or supervised conditions,  are also envisaged for Zones 3,6,7 and 8, giving a total of  5,994000ha in all.)    This implies an emphasis on the extraction of non-timber forest products with some selective felling of mature millable timber species under strictly controlled conditions.

Forest removal for homesteads, food and gardens should be limited to that required for home consumption, and not exceed 2ha of an individual's total holdings; slash and burn agriculture should be completely prohibited, as such leads to a rapid deterioration in land and forest quality.

Areas with significant, but still relatively sparse rural populations have been zoned in this land use category.  They include SOTER land units BO-0010,-14,-15,-17,-18,-19,-21,-40,-41,-46,-47,-48 and BO-0049.  These are found on different landscapes throughout the region.  Their soils are uniformly poor.  Ferralsols (Oxisols and Ultisols) predominate that have low pHs (mean topsoil pH 5.0); these are poorly supplied with plant nutrients and have high exchangeable aluminum levels (mean topsoil level = 2.0 cmol(+)/kg.  Topsoil organic matter levels are moderate (mean topsoil C% = 1.09), but are intimately linked to the little fertility the soils have which is dependent on the forest - soil carbon cycle..

Indiscriminate forest removal on these lands, and in fact over most of the region, can only lead to massive soil water erosion and, or soil chemical degradation, as has already been demonstrated in the Cobija region where lands are being cleared for ranching.  It should also be noted that  their low pH and high aluminum levels indicate potentially toxic conditions for many crops which can only be over-come by heavy, and patently uneconomic lime and fertilizer applications.

The forests covering the zone are seres (intergrades) of tropical semi-evergreen seasonal forests with varying heights and botanical characteristics; however most are naturally endowed with Brazil nut and, or natural rubber trees and to a lesser extent with millable timbers.  Properly cared for, these forests could provide the existing rural population with a reasonable standard of living, and provided that technology is developed to upgrade their productivity over time, and that rural people learn to produce most of their own food supplies, they could support an expanding population.  Clearly a holistic approach is needed to ensure the well being of rural peoples and  to provide for their material and sociological well-being.

From a sustainable production point of view, high priority should be given to:

a)  The establishment of better marketing circumstances for forest products.

b)  Research on developing ways and means of enhancing the extractive value of the native forests in terms of improving the production of both non-timber extractive products and also timber products.

c)  Research on the development of sustainable systems to enable rural people produce most of their own balanced food supplies.

a)    With respect to the marketing of  forest products, this would include the rationalization and relocation of processing facilities for both non-timber and timber products.  Further it would involve the establishment of goodwill and fair pricing arrangements with both the overseas buyers and the rural collectors of the forest products.   The unilateral declaration of a type of processors cartel  to fix prices of Brazil nuts by the Riberalta Brazil nut processors in late 1992, did nothing to improve the image of the local people, and resulted in a virtual  collapse of a traditionally stable market for processed Brazil nuts.  Further, the fluctuating prices paid by processors to rural collectors during the collection season, often results in considerable disillusionment on the part of collectors, and is counterproductive for the industry.

2) and 3)    With respect to research, this needs to be clearly focused on both the need to improve the productivity of the existing forests and at the same time, on the development of sustainable systems for the production of balanced food supplies for rural people that will not be too time consuming in terms of labor.  Neither objective will be achieved without a clear understanding of the soil and its inter-relationship with the native forests.  Both objectives will involve the setting up of much more sophisticated research facilities than those presently established in the region.  Such will include the establishment of  adequate soil and plant pathological laboratories, and the hiring of properly trained personal.

The needed research would be greatly enhanced by a significantly more detailed appraisal of the soil and forest resources of the region.  Further, as sustainability of food crop production must rely on the timely identification of soil fertility and plant pathological problems, and as the enhancement of forest productivity will inevitably involve the production and planting of tree crop seedlings with adequate fertilization, the creation of laboratory facilities  and the training of scientists to run them, must be regarded as a  priority.  Apart from the establishment of adequate research  facilities, the hiring of trained scientists to achieve meaningful results will probably imply the creation of long-term technical assistance arrangements.  The existing complex of organizations involved with agricultural extension activities would greatly benefit from adequate research; the latter are a necessary link in the chain of technology transference to rural peoples.

6.  MANAGED FOREST RESERVES

An area of approximately 486,000ha along the northern Bolivian border with Brazil, which represents various landscapes found in the region including hills, high peneplains and low peneplains with a complex of soil-forest combinations, has been zoned as a reserve for managed forests.  These include SOTER units BO-0032,-33 and -35.  The zone is sparsely populated and relatively untouched in-so-far-as timber extraction is concerned, although it is affected by non-timber extractive activities for Brazil nut and rubber tapping.  Most of the latter activity is carried out by Brazilian seasonal workers, who are not always interested in looking after the renewable resources  they exploit.

The soils of the region are similar to those described for the Cobija region; they are mainly Ferralsols (Oxisols and Ultisols), and are inherently poor.  Their average topsoil pH levels are about 5.3, they are high in exchangeable aluminum, 1.7cmol(+)/kg, although this problem diminishes with depth, and they are poorly supplied with most other plant nutrients.  Like most of the soils of the region as a whole, the forest-soil organic cycle is fundamental to sustained forest production.

The predominant forest covering the region is a medium height, tropical semi-evergreen seasonal forest, with approximately 50-70% of its emergents dry season deciduous, although taller semi-evergreen seasonal forests cover approximately 146,000ha of the area.  Smaller areas of lower forests, mainly weak hydroseres, are found growing on soils with impeded drainage, and are scattered throughout the region.  The forests have distinct floristic variations, and are well representative of much of the northern forests.

It is envisaged that the region will be managed as a forest reserve, and that a center for forestry research and management will eventually be established to find ways and means of maintaining and improving the productive capacity of the forests for both non-timber and timber products.  This center would integrate its activities with other research initiatives for the region as a whole, and especially those focused on improving the bulk of the region zoned as extractive reserves.

7.  CRITICAL WATERSHEDS, HILLY AREAS

The hilly area along the eastern portion of the northern border of Pando has been zoned as a critical watershed area.  It includes SOTER units BO-0034 and BO-0037, which cover approximately 512,000ha.  It might be noted that a part of the area zoned as a managed forest reserve, SOTER unit B0-0035, and part of the region zoned as a fauna reserve, SOTER unit BO-0038, would otherwise have been included as part of this zone.

The hills of the region are not high (200 -250m median elevation), but are characterized by their steep slopes.  They have been dissected from their original elevated plane surfaces by natural geological erosion processes over a long time.  Low fertility Ferralsols (mainly Ultisols and some Oxisols), predominate in the region; the Ultisols are highly susceptible to erosion, having light textured easily erodable topsoils over heavy subsoils.

Soil degradation by water erosion, with its dire consequence of silting up local rivers, can only be avoided by maintaining the present forest cover on these lands.  This implies a strict control on logging, although non-timber extractive activities could be allowed  on a seasonal basis.  The region is not suitable for permanent settlement, as such would invariably lead to land clearing and serious soil erosion.

8.  CRITICAL WATERSHEDS, FLOODED AREAS

Most of the alluvial lands along the extensive river systems of the region are subject to severe annual floods, and are quite unsuitable for close settlement.  Exceptions can be found where occasional higher lands are not affected by floods, or rarely affected, but such areas are small in extent.

Hydrologically speaking, the existing flood plain areas provide expansion zones to ameliorate peak flow rates of rivers, and the flood forest cover of such plays a critical role in reducing water erosion.  Without such mechanisms, existing townships sited in riberine areas, including Riberalta and Guayaramerim in the Beni Department, would simply be washed away.  Consequently, it is important that the river flood plain area be protected from extensive deforestation, including the establishment of seasonal pastures.

Many of the alluvial land along the rivers contain substantial areas of rubber trees which are still being tapped seasonally by local people, and providing a source of useful income.  In this respect, it is worth noting that improvements in the technique of coagulating rubber could make rubber collection a more viable source of income for people precariously established in riverine areas.  Further, selective extraction of some timber species, including mahogany which appears to tolerate soils with seasonally flooded and, or impeded drainage conditions, and not the wholesale removal practiced by logging companies, would probably not cause serious harm to such regions.

SECTION IV

SUMMARY AND CONCLUSIONS

Climate

The climate of the Pando Department is adequate for most tropical tree crops that can withstand a 3 to 4 month dry season, and that are not adversely affected by the often severe fluctuations in temperatures caused by the "surazos" or cold fronts.  The latter are characteristic of the cool "winter" season when temperatures may drop to six degrees centigrade or less!  Clearly the wet summer period is suitable for growing most short term tropical annual crops.  Annual rainfall distribution is slightly higher in the western sector of the region, about 1,800mm, and grades to about 1,600mm in the east.

Landscapes

The region which covers approximately 6,319,000 ha, is predominantly a lowlands plains area;  high (200-300m),  peneplains in the west grade unevenly to lower peneplains and plains (100-150m), in the east.  In the north-west the higher peneplains are strongly dissected, forming hill formations. The area is subdivided west to east by easterly running rivers, which join northerly running rivers in the east. All have substantial flood plains subject to annual floods. The area is covered by tropical semi-evergreen seasonal forests and their hydroseres, with the exception of about 19,000 ha of predominantly wet savannas in the south-east sector.

Soils

It was estimated that  3,308,762 ha, or approximately 52% of the region has soils with slopes less than 8%, and that are neither flooded nor poorly drained. This statistic, which at first glance might indicate the extension of potentially suitable lands for agricultural or agro-pastoral purposes from a gross physical perspective, does not take to account other soil physical limitations including soil textural problems such as sandy or gravely conditions, water holding capacities or root restricting layers. Further, it obviously does not take soil fertility  to account.

The examination of the soil fertility conditions of the latter soils indicated that approximately 70% have potentially severe aluminum toxicity problems in their subsoils (greater that 55% Al saturation of cation exchange capacity), and 51% have the same problem with their topsoils.  Further,  most of these soils are  poorly supplied with the major plant nutrients. -Their overall nutrient status is clearly associated with topsoil organic matter levels;  the latter is built up over many years as a result of the forest organic matter cycle.

It is sad, but of great significance for the planning of effective land usage in the Pando Department, that no extensive areas of inherently fertile soils were found in the region.

Land use

As an overall estimate, about  80,000 ha of the department, has been affected by deforestation in recent years for both food crop and pasture production for semi-intensive cattle ranching.  About one third of this total represents forest clearings in various stages of regeneration.  Notwithstanding, deforestation in the vicinity of  Cobija, has increased dramatically over the past 7 years.

Between 1986 and 1990, satellite imagery showed that the area cleared in the vicinity of Cobija increased by 56% (27,680 ha to 43,080 ha).  (Refer PC Users Packet, Section 2, IDRISI Files, attached for maps).  From what may be seen in the field, it is clear that deforestation continues to proceed apace in these regions, and that estimates of deforestation based on the 1990 satellite imagery, are too conservative.  Unfortunately,  there is very little evidence to show that the resulting agricultural and, or pastoral activities have been successful in either region, despite the diligent attempts of both official and non-governmental organizations to provide technical assistance.

The predominant land use of the region is, and from the analysis of its land resources should remain, the extraction of Brazil nuts and rubber tapping; over 50% of the total region is covered by forests with varying Brazil nut tree population densities, and a larger percentage still, has native rubber.  However, in recent years due to fluctuating rubber and Brazil nut prices, there has been a marked increase in the extraction of the more valuable timber species, particularly mahogany and cedar.  The typical forest sound in the region is no longer the whistle of the "seringeiro" birds or the chatter of monkeys, but the high-pitched whine of chain saws.

Land Use Zoning

A Land Use Zoning map has been drawn up on the basis of the research on the  existing soil and forest resources of the Department, to provide a realistic guide for future land usage of the region.   It is clear that Pando is blessed with unique renewable forestry resources;  unfortunately the same can not be said about its soils, which are generally inherently infertile and unable to support intensive  agricultural usage without the input of large and costly soil amendments including lime and fertilizers.   Consequently, the region has been zoned to take advantage of the regions natural advantages.

Nature reserves -  the center of origin of Brazil nuts, cacao and rubber.

It should be noted that the areas potentially zoned as natural reserves should be regarded as areas for the careful preservation of the regions typical flora and fauna.   In the special case of the natural reserve for flora, that area can  be regarded as a typical part of the "center of origin" of many of Amazonia's natural forest tree products including Brazil nuts, cacao and rubber.   Its unique biodiversity and genetic resources  should be regarded  as a treasure reserve not only for future generations of Bolivians, but also for all humanity;  international attention should be focused on this zone and finance sought for its proper maintenance.   Logging in the region, and forest clearing must be stopped immediately.

CONCLUSIONS

From an agro-ecological viewpoint the region, which still has most of its original forests "physiognomically" intact except in the environs of its major population centers, fits the classical picture of an Amazonian forest growing on poor soils, in a fragile balance with its organic cycle. Accelerated deforestation without sound crop husbandry supported by adequate liming and fertilization, can only lead to soil degradation and eventual economic and ecological  failure.

Agro-forestry, (perennial tree crop and timber cultivation), might have a chance to succeed if cheap supplies of lime and fertilizer become available, and adequate markets found for produce.   However, international experience over the past 50 or so years, indicates that agro-forestry must largely be based on non-native (or exotic) crops, if it is to be successful.  This is because tree crops grown near their centers of origin invariably are affected by indigenous pests and diseases (Pers. Comm., Prof. J.Purseglove, ex-curator of the Singapore Botanical Gardens).  -The case history of rubber grown at Fordlandia in Brazil in the 1930's(affected by South American Leaf Blight), and during the last decade, that of cacao grown in the State of Rondonia just over the Bolivian border in Brazil (affected by Witches Broom), are but two of many instances of  failures to establish native tree crops.   The establishment of  viable agro-forestry systems  requires careful research, and very few instances of the successful economic implementation of such have been achieved to date, and only at the expense of the virtual loss of native forests.

It is clear that the present extractive activities could be expanded quite significantly and made more  efficient with proper organization.  However policies to do so, including the better organization of extractive reserves and processing and marketing facilities, must always bear in mind the inherently poor soil resources of the region and consequently the need to preserve  an adequate forest cover to avoid land degradation, with its high economic and human costs.  The position of the Pando department is unique, in that if has been blessed with some of Amazonia's most productive non-timber extractive forests.    Responsible land usage based on the more effective use and maintenance of these resources could ensure the future prosperity of the Department;  this will not be achieved if the present trend of accelerated deforestation continues.

Finally, it is in the interest of both the peoples living in Pando and Bolivia as a nation, to see the proper establishment and maintenance of the zoned natural reserves for Amazonian flora and fauna.   This will provide an opportunity for establishing not only a progressive and potentially financially rewarding "eco-tourism" industry, but also for creating a world recognized "resource for humanity", with its many collateral benefits.

REFERENCES

Ahlfeld, F. and L. Branisa. 1960.  Geología de Bolivia. Instituto Boliviano de Petróleo.  Ed. Don Bosco, La Paz, Bolivia.

Alvim, P.T. 1978.  Perspectivas de producáo agricola na regiao amazónica.  Interciencia 3(4): 243-249.

Alvim, P.T. 1979.  Agricultural production potential of the Amazon region.  In: P.A. Sanchez and L.E. Tergas (eds.).  Pasture production in acid soils of the tropics.  CIAT, Cali, Colombia. p. 13-23.

Budowski, G. 1976.  La conservación del medio ambiente. Conflicto o instrumento para el desarrollo? Ciencia Interameri-cana 17 (1): 2-8.

Buol, S.W., P.A. Sanchez, R.B. Cate and M.A. Granger. 1975.  Soil fertility classification.  In: Bornemiza, E. and Alvarado (eds.) Soil management in Tropical America, NCSU, Raleigh, USA, p. 126-141.

Carrera, W.M. 1976. Estudio de la Sub-estación experimental de Riberalta, Beni, unpublished report, MACA, Bolivia, 30p.

Cochrane, T.T. and D.M.G. de Souza. 1985.  Measuring surface charge characteristics in oxisols and ultisols.  Soil Science, 140 (3): 233-229.

Cochrane, T.T. 1973. The land use potential of Bolivia: a land systems map. Ministry of Overseas Development, F.C.O. London, England. 827p.

Cochrane, T.T. and R. Barber. 1993. Análisis de suelos y plantas tropicales. British Mission in Tropical Agriculture, Santa Cruz, Bolivia.  ODA, FCO, London 31 chpts. + appendices.

Cochrane, T.T. and P.G. Jones. 1981, Savannas, forests and wet season potencial evapotranspiration in tropical South America Trop. Agric. 58 (3): 185-190.

Cochrane, T.T; J.G. Salinas and P.A. Sanchez. 1980. An equation for liming acid mineral to compesate crop Al tolerance. Trop. Agric. 59: 133-140.

Cochrane, T.T. and P.A. Sanchez, 1982. Land resources, soils and their management in the Amazon Region; a State of Knowledge report. In: S.B. Hecht (ed.1. Amazonia: Agriculture and land use research). CIAT, Cali, Colombia. p 137-210.

Cochrane, T.T, L.G Sanchez, L.G. Porras, L.G. de Azevedo and C.L Garver. 1985. Land in Tropical America.  CIAT, Cali, Colombia, EMBRAPA-CPAC, Planaltina. D.F. Brsil, ISBN 84-89206-39-2 vols.+ maps.

CONET, 1978a. Estudio de Recursos Naturales Cobija-Puerto Heath. Tomo I. Consultores Económicos y Técnicos "CONET"/CORDENO, Cobija, Bolivia 56 pp. + 3 anexos.

CONET, 1978b. Estudio de Recursos Naturales Cobija-Puerto Heath. Tomo III Inventario de especies forestales. Tomo III. 246 pp. Consultores Económicos y Técnicos/CORDENO. Cobija-Bolivia.

CONSA Consultores Asociados y HARZA Ingeneering company International, 1983. Proyecto de Colonización Puerto Hearth-Extrema. Diagnóstico general (informe Preliminar). Ministerio de Asuntos Campesinos y Agropecuarios, Instituto Nacional de Colonización. 101 pp. + 10 anexos.

DHV Consultants 3.V. 1991.  Agro-ecological and socio-economic studies in the Brazil nut region of Northern Bolivia. Terms of reference and implementation phase.  DHV Consultants BV. P.O. Box 1399, NL-3800 BJ Amersfoort.

Eastman, J.R. 1992. IDRISI Version 4.0. Clark University, Graduate Schoool of Geography and UNEP.

Falesi, I.C. 1972. O estado atual dos conhecimentos sobre os solos da Amazonia brasileira. Bol. Tec. IPAN 54; 17-67.

Falesi, I.C. 1976. Ecossistema de pastagem cultivado na Amazonia brasileira. Bol. Tec. CPATU,EMBRAPA, Belén, Brasil, 193p.

FAO. 1990. FAO-UNESCO Soil map of the world revised legend. World Soil Resources Report 60. FAO, Rome. 11 ap.

FAO. 1990a. Guidlines for soil description. 3rd. Ed. FAO, Rome.

FAO-ISRIC. 1991. FAO-ISRIC Soil Database (SDB2). World Soil Resources Reports 64 (1989). 90 p.

FAO-ISRIC. 1993.  Global and national soils and terrain digital databases (SOTER) : procedures manual. (ed.: V.W.P. van Engelen et al.) International Soil Reference and Information Centre, Wageningen, Holland. ISBN 90-6672-051-4.115p.

FAO-UNESCO. 1971. Soil map of the world 1:5.000,000. Volume IV South America. Paris, France, 189 p. and map.

FAO-UNESCO. 1976. Legend for the Soil map of the world. UNESCO, Paris, France.

Goodland, R.J.A., H.S. Irwin and G.Tillman. 1978. Ecological development for Amazaonia. Ciencia e cultura 30 (3): 275-289.

Goodland, R.J.A. and H. S. Irwin. 1975. Amazon jungle: green hell to red desert? Elseuier, Amsterdam. The Netherlands.

Goury, P. 1961. The tropical world; its social and economic conditions and its future status. 3rd. ed. (Translated by E.D. Laborde) Longmans, London, England.

Hargreaves, G.H. 1972. The evaluation of water deficiencies. Age of changing priorities for land and water. irrigation and drainage speciality conference. ASCE, Spokane, Washington p. 273- 290.

Hargreaves, G.H. 1977. World water for agriculture (USAID Contract Nº AID/Ta-c-1103) Utah State University, Utah, USA. 1787p.

Hargreaves, G.H. E.C. Olsen and J. Vanegas. 1978. Moisture precipitation probabilities, potential evapotranspiration requirements and climatic classification for Peru. Utah State University. Utah, USA 79p.

Hargreaves, G.H. 1989. Accuracy of estimated reference crop evapotranspiration. Journal of Irrigation and Drainage Engineering. 115 (6) 1000-1007.

Hancock, J.K,R.W Hill and G.H. Gargreanes. 1979. Potential Evapotranspiration and precipitation deficits for tropical America CIAT, Colombia, 398p.

Iporre-B, J. 1984. Estudio de suelos área del proyecto Vaca Diez. Programa de Desarrollo Integral de la Amazonía, Bolivia. OEA Ministerio de Planeamiento y Coordinación, Bolivia. Un published report 60p. appendices and map.

Iporre-B, J. 1985a. Estudio de suelos, área del proyecto Santa Rosa del Abuna. Programa de Desarrollo Integral de la Amazonía, Bolivia. OEA. Ministerio de Planeamiento y Coordinación, Bolivia. Unpublished report. 60p. appendices and map.

Iriarte-A.R, Calero-CH-E, Hinojosa-S,P and R. Torres-S. 1976. Clasificación taxonómica y por capacidad de uso de tierras. Area: Riberalta, Guayaramerin, Cachuela Esperanza, Provincia Vaca Diez, Departamento del Beni, Bolivia, Un published report, Departamento de Suelos MACA, Bolivia. 184 p and maps.

Irion, G. 1978. Soil infertility in the Amazonian rainforest Naturwissenschaften 65: 515-519.

ISRIC. 1990a. SOTER Procedures manual for small-scale map and database compilation. Eds. V.W.P. Van Engelen and J.H.M. Pulles. Working paper and preprint 90/2, ISRIC, Wageningen, The Netherlands, 60p.

ISRIC, 1990b. Proceeding of the international workshop on procedures manual arevisions for the Global Soils and Terrain Digital Database Wageningen 24-26 April 1990. Ed. N.H. Batjes. Working pape and preprint 90/5, ISRIC, Wageningen, The Netherlands, 25p.

ISRIC. 1991. The SOTER manual, Procedures for small scale digital map and data base compilation of soil and terrain conditions. Eds.V.W.P. van Engelen and J.H.M. Pulles. Working pape and preprint 91/3, ISRIC, Wageningen, The Netherlands, 91p.

ISSS. 1986a. Procedings of an international workshop on the structure of a digital international soil resources map annex database. Eds. MF. Baumgardner and L.F. Oldeman. SOTER report 1, ISSS, Wageningen The Netherlands, 139p.

ISSS. 1986b. Project proposal "World soil and terrain digital database at a scale 1:1M (SOTER) Ed. M.F. Baumgardner, ISSS, Wageningen, The Netherlands, 23p.

ISSS. 1987. Proceding of the second international workshop on a global soil and terrain digital database, 18-22 May 1987, at UNEP Nairobi. Ed. R.F. van de Weg. SOTER report 2, ISSS, Wageningen. The Netherlands, 47p.

Jones, J. 1991. TOSCA, Version 1.0 Clark University, Graduate School of Geography.

Leyton, F. y J. Pacheco, 1989. Geología del Cuaternario-Terciario aflorante en el Río Madre de Dios. Sociedad Geológica Boliviana. YPFB. Memorias Bodas de Plata, Publicación Especial.

Macias. M. 1985. Estudio de suelos área del Proyecto Ballivian norte. Programa de Desarrollo Integral de la Amazonía, Bolivia. OEA. Ministerio de Planeamiento y Coordinación. Bolivia. Unpublished report. 59p. appendices and map.

Macias, M. 1984. Estudio de suelos área del Proyecto Palma Africana. Programa de Desarrollo Integral de la Amazonía, Bolivia. O.E.A.Ministerio de Planeamiento y Coordinación. Bolivia Un published report- 60p. appendices and map.

Povoa. A.F.R. 1978. Manual for dryland and irrigated crop production: Monthy climatic data tables for Latin America. IIC, Utah State Universtiy, Utah. 98p.

Reis, A.C.F. 1972. O Impacto Amazonico na civilizacao brasileira; a transamazonica e o desafio dos trópicos, Ed. Paralelo, Río de Janeiro, Brasil.

Remmelzwaal, A. 1990. Classification of land and land use, first approach. FAO Rome (un published).

Sainz, A.J. Mercado and F. Camacho. 1984. Proyecto Desarrollo Agrícola "Provenir", Prov. Nicolas Suarez, Departamento Pando, Bolivia. Ministerio de Planeamiento y Coordinación. MACA, COTESU La Paz, Bolivia. 105 PP + anexos.

Salm, H. and M.Marconi. 1992. Reserva nacional amazonica Manuripi-Heath. Programa de Reestructuración, fasc.11 LIDEMA, La Paz, Bolivia (Unpublished report).

Sanchez, P.A. 1977. Advances in management of oxisols and ultisols in Tropical South America, In: Proc. Int. Seminar on Soil Environment and Fertility Managenent in Intensive Agric. (Tokyo, 1977). Soc. of the Sci. of Soil and Manure. Tokyo, Japan, p. 535-566.

Serrao, E.A.S.,I.C;, J.B Veiga and J.F. Texeira Neto, 1979. Productivity of cultivated pastures in low fertility soils of the Amazon of Brazil. In: P.A. Sanchez and L.E. Tergas (eds), Pasture production in acid soils of the tropics CIAT, Cali, Colombia, p. 195-226.

Seltzer, J. 1967. Impossibilidade do uso racional do solo no alto Xingú, Mato Grosso, Rev. Brasileira Geogra. 21 (1): 102-109.

Shields, J.A. and D.R. Coote. 1988. SOTER procedures manual for small-scale map and database compilation. Working paper and database compilation. Working paper and preprint 88/2, ISRIC, Wageningen, The Netherlands, 144 p.

Shields, J.A. and D.R. Coote. 1989. SOTER Procedures manual for small-scale map and database compilation including proposed procedures for interpretation of soil degradation status and risk. Internal report ISRIC, Wageningen, The Netherlands.

Schubart, H.O.R. 1977. Criterios ecológicos para o desenvolvimento agrícola das terras firmes da Amazonia. Acta Amazonica 7 (4): 559-567.

SNA, Servicio Nacional de Aerofotogrametría 1987. Estudios de recursos naturales renovables a nivel de reconocimiento Pardo,Bolivia. Tomos I y II CORDENO, FAB. La Paz. Bolivia. I:127- pp + anexos II: 55 pp. + anexos.

Toledo, J.M. and V.a. Morales. 1979. Establishment and Management of improved pastures in the Peruvian amazon. In: P.A. Sanchez and L.E. Tergas (eds.), Pasture production in acia soils of the tropics CIAT, Cali, Colombia. p17-194.

Tosi, J.Jr. 1976. Desarrollo forestal del trópico americanoa frente a otras actividades económicas. In: Reunión Int. sobre sistemas de producción para el trópico americano informes de conferencias 41, IICA, Lima,Peru. ppII-F-1..13.

UNESCO. 1973.  International classification and mapping of vegetation, UNESCO, Paris.

Van Reeuwijk, L.P. (ed). 1987. Procedures for soil analysis. 2nd. Edit. Tech Paper Nº 9 ISRIC, Wageningen, The Netherlands. 19 chpts + appendices.

Van Wambeke. 1981. Calculated soil moisture and temperature regimes of South America, SMSS, SCS, USDA, Wageningen, D.C. USA 25p and maps.

TABLES.

I.	METEOROLOGICAL DATA SETS

II.	REGIONAL LANDFORMS

III.	REGIONAL LANDSCAPES

IV.	REGIONAL NATIVE VEGETATION CLASSES

V.	CROSS-TABULATION OF REGIONAL LANDSCAPE  X

NATIVE VEGETATION CLASSES

VI.	REGIONAL SOIL CLASSES

VII CHEMICAL PROPERTIES OF THE FLAT, WELL DRAINED, NON

FLOODED  SOILS OF THE REGION.

TABLE I.              METEOROLOGICAL DATA SETS

COBIJA      BO-001     LAT 11°1'   LONG 66°44'   ALT 280 m

JAN     FEB    MAR   APR   MAY   JUN    JUL   AUG   SEP    OCT   NOV   DEC  ANNUAL

TEMP    25.6      25.3      25.1     24.7    24.1     22.9     22.5    23.1    25.6    25.9     25.9     25.5     24.7

SUNP     25.0      31.0      31.0     32.0    38.0     55.0     51.0    47.0    33.0    31.0     26.0     27.0     36.0

RADI     360.0    400.0    380.0   355.0  345.0   390.0   385.0  410.0  380.0  395.0   370.0   380.0   379.0

RAIN     233.0    222.0    213.0   192.0  80.0     25.0     25.0    36.0    94.0    165.0   192.0   251.0   1728.0

PETH     113.0    112.0    117.0   105.0  104.0   110.0   111.0  120.0  115.0  124.0   113.0   118.0   1363.0

RDEF     -120.0   -110.0   -96.0   -87.0   24.0     85.0     86.0    84.0    21.0    -41.0    -79.0   -132.0  -365.0

RR75      151.0    143.0    137.0   123.0  45.0     7.0       7.0      15.0    55.0    104.0   123.0   163.0

MAI       1.34      1.28      1.17     1.17    0.44     0.07     0.07    0.12    0.48    0.84     1.09     1.38

RIBERALTA   BO-002     LAT 11°0'   LONG 66° 5'   ALT 172 m

JAN    FEB    MAR   APR   MAY   JUN    JUL   AUG    SEP    OCT    NOV    DEC  ANNUAL

TEMP    26.8     26.7     26.8      26.6     26.1     25.6     25.6    27.5      28.6    27.9     27.6      27.0     26.9

SUNP     22.0     31.0     29.0      22.0     38.0     54.0    49.0     49.0     33.0     29.0     26.0      23.0     34.0

RADI     340.0   400.0   370.0    295.0   345.0   385.0   380.0   420.0   380.0  385.0   370.0    350.0   368.0

RAIN     242.0   247.0   256.0    148.0   81.0     30.0     13.0     20.0     59.0    131.0   157.0    265.0   1649.0

PETH     109.0   116.0   119.0    91.0     109.0   116.0   119.0   137.0   123.0  127.0   117.0    113.0   1397.0

RDEF     -133.0  -131.0  -137.0  -57.0    28.0     86.0     106.0   117.0   64.0    -4.0      -40.0    -152.0  -252.0

RR75      157.0   160.0   167.0    92.0     46.0     11.0     0.0       4.0       31.0    81.0     98.0      173.0

MAI       1.44     1.39     1.40      1.01     0.42     0.09     0.00     0.03     0.25    0.63     0.84      1.53

GUAYARAMERIN  BO-003     LAT 10°48'  LONG 65°22'   ALT 172 m

JAN   FEB    MAR   APR   MAY   JUN    JUL   AUG    SEP   OCT    NOV    DEC  ANNUAL

TEMP    26.2     26.5     27.0      26.7     26.1     26.1     26.1    25.9     27.7    28.5     26.3     26.2      26.6

SUNP     21.0     29.0     29.0      24.0     42.0     68.0    64.0     49.0     33.0    29.0     21.0     23.0      36.0

RADI     330.0   390.0   370.0    305.0   365.0   435.0   435.0  420.0   380.0  385.0   330.0   350.0    375.0

RAIN     242.0   172.0   226.0    137.0   46.0     23.0     0.0      4.0       56.0    104.0   212.0   227.0    1450.0

PETH     105.0   112.0   119.0    95.0     115.0   133.0   138.0  132.0   121.0  129.0   101.0   111.0    1411.0

RDEF     -137.0  -60.0    -106.0  -43.0    69.0     110.0   138.0  128.0   65.0    25.0     -111.0  -116.0  -39.0

RR75      157.0   109.0   146.0    85.0     22.0     6.0       0.0      0.0       29.0    62.0     137.0   147.0

MAI       1.50     0.97     1.22      0.90     0.19     0.05     0.00    0.00     0.24    0.48     1.35     1.32

PUERTO MALDONADO  BO-004     LAT 12°36'  LONG 69°12'   ALT 200 m

JAN    FEB    MAR   APR   MAY   JUN   JUL   AUG   SEP   OCT    NOV    DEC  ANNUAL

TEMP    26.8     26.7     26.3     26.5     25.8     25.0     24.2    27.2    28.0    27.2     27.2     26.8     26.5

SUNP     14.0     15.0     13.0     13.0    12.0      12.0     11.0    11.0    14.0    13.0     15.0     13.0    13.0

RADI     272.0   279.0   250.0   226.0   189.0   180.0   178.0  198.0  246.0  257.0   279.0   265.0   235.0

RAIN     287.0   248.0   287.0   176.0   143.0   37.0     60.0    54.0    80.0    112.0   137.0   304.0   1925.0

PETH

RDEF

RR75      198.0   170.0   198.0   119.0   96.0     20.0     36.0    32.0    51.0    73.0     91.0     210.0

MAI       1.77     2.05     1.71     0.65     0.30     0.26     0.30    0.22    0.36    0.91     1.14     1.60

RURRENABAQUE  BO-005     LAT 14°25'  LONG 67°35'   ALT 227 m

JAN    FEB    MAR   APR   MAY   JUN   JUL   AUG   SEP   OCT   NOV    DEC  ANNUAL

TEMP   26.8     26.9     26.5     24.0     24.6     23.0     22.6     24.6    26.8    27.3    27.5    27.2     25.6

SUNP    30.0     47.0     35.0     29.0     42.0     53.0     54.0     50.0    38.0    37.0    35.0    38.0     41.0

RADI    410.0   495.0   405.0   325.0   345.0   360.0   375.0   405.0  400.0  435.0  440.0  460.0   405.0

RAIN    275.0   279.0   206.0   111.0   104.0   120.0   93.0     56.0    73.0    112.0  150.0  228.0   1807.0

PETH    132.0   144.0   129.0   94.0     105.0   102.0   109.0   123.0  124.0  141.0  139.0  149.0   1493.0

RDEF    -143.0  -135.0  -77.0   -17.0    1.0       -18.0   16.0      68.0   51.0    29.0    -11.0   -79.0    -314.0

RR75     180.0   183.0   132.0   67.0     62.0     73.0     54.0     29.0    40.0    67.0    94.0    147.0

MAI      1.37     1.27     1.02     0.71     0.59     0.71     0.50     0.23    0.33    0.48    0.67    0.99

TABLE II.        REGIONAL LANDFORMS

Landform                           Area in ha.

Floodplains                              1,198,300

Low plains, Alt<150m                 804,100

Low peneplains, Alt<150m       1,165,300

High peneplains, Alt>150m       2,345,300

Dissected  plains                          363.000

Hills, medium height                     442,000

Total surface (including

rivers and lagoons)                    6,318,900

TABLE III.        REGIONAL LANDSCAPES

Landscape                              Area in ha.

Floodplains                                1,198,300

Plains, Alt<150m                          809,000

Low peneplains, Alt<150m        1,282,400

High peneplains, Alt>150m        2,160,500

Hills                                              752,500

Poorly drained lands                     825,700

Savanna lands                                18,600

Total surface (including

rivers and lagoons)                       6,318,900

TABLE IV.      REGIONAL NATIVE VEGETATION CLASSES

(SOTER classes adapted from UNESCO, 1973)

FOREST VEGETATION

Code             Description                                            Area in ha.

IA2E           Tropical semi-evergreen seasonal

forest, 50-100% of emergents dry

season deciduous

Forests on well drained, non-flooded soils:

IA2ET      AS IA2E, but tall: emergents over 40 m.           1,021,400

IA2EM     AS IA2E, but medium: emergents 30-40 m.      3,265,400

IA2EL      AS IA2E, but low: emergents less than 30 m.

Forests on imperfectly drained or flooded soils:

IA2EMG   AS IA2E, but medium, drainage imperfect         130,700

IA2ELG    AS IA2E, but low, drainage imperfect                569,700

IA2ELH    AS IA2E, but low, drainage poor                       204,800

IA2EMF   AS IA2E, but medium, annually flooded             444,000

IA2ELF    AS IA2E, but low, annually flooded                      84,800

IAS           Swamp forests: low with graminoid species        579,500

SAVANNA VEGETATION

Code                  Description                                           Area in ha.

VB6      Poorly drained savannas                                           18,600

Total Remaining Native Vegetation:                         6,240,000

Note.  (All estimates include areas taken up by rivers,  lagoons

and swamps.)

TABLE V.     CROSS-TABULATION OF REGIONAL LANDSCAPE

x    NATIVE VEGETATION CLASSES

TABLE VI.      REGIONAL SOIL CLASSES ACCORDING TO THE

FAO-UNESCO LEGEND AND SOIL TAXONOMY

FAO-UNESCO                        SOIL TAXONOMY

Legend           Area in ha.               Class        Area in ha.

FLDGL                21,939                 EQTAA              21,939

FLDIN                 48,565                 EVTXA              48,565

FLEIN                514,775                EVTAA            514775

GLDIN               108,394                EVTAQ            428,883

GLDVA              214,688                EVTAX            106,900

GLEIN                112,700               UDLPE             18,044

PLDPF                 18,044                OQAAA           514,118

FRXVG              464,954                OQAXA           179,245

FRHVG              486,176                OQAHU             42,925

FRHVA              742,904                OUAAQ             31,735

FRRVA            2,483,394               OUAAA         2,359,760

FRXVA              526,624                OUARH            102,766

LXFVA              475,743                OUAXA              19,085

ODAAA            438,648

UUHAA            398,512

UUHXA            338,279

UUKAA            302,283

UHKAA            305,094

UDAAA            153,146

SUMMARY

FL (fluvisols)           585,279               E (entisols)      1,121,062

PL (plansols)             18,044               U (ultisols)       1,515,358

GL (gleysols)           535,782               O (oxisols)      3,682,480

FR (ferralsols)       4,704,052

LX (lixisols               475,743

TOTAL:       6,318,900

TABLE VII.   CHEMICAL PROPERTIES OF THE FLAT, WELL

DRAINED, NON FLOODED SOILS OF THE REGION.

TOTAL AREA OF THESE LANDS:  3,308,782 ha.

Percentages of the area.

pH            Low < 5.4            Med 5.4 - 6.5       High > 6.5

topsoil            79                              21                  0

subsoil            70                              30                  0

Carbon        Low < 0.6%        Med 0.6 - 3%        High > 3%

topsoil              4                                 96                  0

subsoil             79                                21                  0

Phosphorus    Low < 3ppm       Med 3 - 6ppm        High > 6ppm

(Olsen)

topsoil               48                              52                   0

subsoil               94                                6                   0

Calcium       Low < 0.4         Med 0.4 - 4            > 4    (cmol(+)/kg of exchangeable

cations)

topsoil            26                          74                       0

subsoil           67                           33                       0

Magnesium     Low < 0.2      Med 0.2 - 0.8       High > 0.8

topsoil           18                             68                      14

subsoil           28                             76                      6

Potasium      Low < 0.15        Med 0.15- 0.3       High > 0.15

topsoil            32                            57                       11

subsoil            79                            10                       11

Sodium        Low < 0.1             > 0.1

topsoil            72                          28

subsoil            78                          22

Aluminium     Low < 1           Med 1 - 1.5         High > 1.5

topsoil               22                       27                       51

subsoil               21                       20                       59

TABLE VII  (continued)

(cmol(+)/kg)

ECEC          Low < 4              Med 4 - 10          High > 10

topsoil             70                       30                         0

subsoil             77                       23                          0

Al Sat. of ECEC.  High >55%

Topsoil               51

Subsoil               70

PC USERS PACKET

README

WELCOME TO SOTER, The World Soils and Terrain Digital Database!

Your PC Users Packet has been subdivided into 3 SECTIONS, each of which contains files for different programs:

SECTION 1.  SOTER4 files.  Please refer to Figure 2 and Appendix II in the report for an overview of the structure of these files, and also to the SOTER Manual ( FAO-ISRIC, 1993).  The files are designed to be used with PARADOX  for Windows or PARADOX 4.O for DOS, and dBASE,  but they may also be used with most other RDBMSs.  The  numeric data may be exported to  most spreadsheet programs including QUATTRO PRO 5.0, and others.   The SOTER4 dBASE files have been included on the diskette (empty, without records), to facilitate users of  dBase.  (With the exception of the PROF.GRP file and other minor files).  The SOTER4 diskette contains 228 files occupying 1,212,362 bytes.  A list of these files is included with APPENDIX  V of the report.

SECTION 2.   IDRISI files.   These include IDRISI image, vector, value and palette files.

The  IDRISI IMAGE files for the SOTER  UNIT maps have been prepared at 3 different pixel scales, the PANDO001C series at the 1ha per pixel scale, the PANDO004 series at the 4 ha per pixel scale and the PAND-100- series at the 100 ha per pixel scale, the latter to speed up the display of maps.  The THEMATIC maps have also been produced at the 100 ha per pixel scale, but can readily be produced using smaller pixels.

NOTE.    When using the COLOR command in IDRISI to display the image files,  if you choose the default answer "n" when asked if you want to "set the display parameters by hand", you may have to press "w" and then the right and left mouse buttons of the mouse at the same time to see the whole image on your screen.  This is because IDRISI will only show the upper left corner of an image when it is too large to fit on the screen with an expansion factor of 1.    -Please check the other commands in the COLOR mode (v..to overlay the vector files on a corresponding image; p.. to display a different palette; x..for interactive query of UTM coordinates and pixel value, and so on...

2.   In addition to the map files of the actual study, files showing DEFORESTATION between 1986 and 1990 for  the Cobija area respectively, have been prepared.

3.   Diskette No.4 of this series contain the original "vector" input files.

Note:	It should be noted that IDRISI facilitates the export of files in a wide variety of formats.  Also note that  "value"  files can be assigned to image files of different scales.

A list of the IDRISI files has been included in APPENDIX V of the report.

SECTION 3.  The FAO-ISRIC  Soil (profile) Database files.

These files have been prepared together with a modified FAO-ISRIC program for their immediate use (refer to Appendix III of the report).   Simply copy the files to a "SDB" directory on your HD, and type SDB2.  The FAO-ISRIC Soil Database manual is available from FAO (World Soil Resources Report No. 64).  A list of these files has been includes in APPENDIX V of the report.

ENJOY SOTER!

Figure 1. The data model.

Figure 2.  The World Soil and Terrain Digital Database (SOTER) attribute data file

system.

Figure 3.  The World Soil and Terrain Digital Database (SOTER) system.